东集 EUIC S

Android Scan Service Interface Configuration

www.seuic.com

| Revising index |  |  |  |
| --- | --- | --- | --- |
| Version | Update date | Revising details | By |
| 1.0 | 21/09/2015 | Create the Chinese version | Liangwei Xu |
| 1.1 | 07/11/2016 | Add barcode leading and trailing space filtering option | Tao Luo |
| 1.2 | 20/03/2017 | Special scan function demo realization | Tao Luo |
| 1.3 | 31/05/2017 | Add the introduction of scanner on/off and non-printable character filter broadcast | Tao Luo |
| 1.4 | 18/07/2017 | Revise the barcode parameters and set the broadcast interface | Tao Luo |
| 1.5 | 03/04/2018 | Add the introduction of start/stop scan broadcast | Tao Luo |

Content

Abstract	3rd

Hardware platform	3rd

Software platform	3rd

Object oriented	3rd

Scan Service Application Setting	4th

Scan sound on/off switch	5th

Vibration on/off switch	5th

Continuous scan on/off switch	5th

Time interval setting	5th

Barcode prefix parameter setting	6th

Barcode suffix parameter setting	6th

Automatically on when power on	6th

End char setting	6th

Barcode transmit method	7th

Barcode broadcast method	7th

Filer for the barcode leading and trailing space	8th

Barcode parameters setting	8th

Scanner activated/disabled setting	8th

Non-printable character filter	9th

Start Scan	9th

Stop Scan	9th

Special Scan Function realization	9th

Continuous light mode setting	9th

# Abstract

SEUIC device has built-in scan configuration and the corresponding setting software. Since some clients are in need of customizing scan service in their own applications, this documents is created to introduce some options about how to set the scan service via interface therefore help better and flexibly use the products.

## Software platform

SDK is supports Eclipse,Android Studio developing tools.

## Object oriented

It’s designed for the developers who would like to customize scan service in their own apps.

# Scan Service Application Setting

Scan service application setting support the below function settings:

- Scan sound on/off switch

- Vibration on/off switch

- Continuous scan on/off switch

- Time interval setting

- Barcode prefix parameter setting

- Barcode suffix parameter setting

- Automatically on when power on

- End char setting

- Barcode transmit method

- Barcode broadcast method

- Filer for the barcode leading and trailing space

- Barcode parameters setting

- Scanner activated/disabled setting

- Non-printable character filter

- Start Scan

- Stop Scan

Customized version: Scan application from 1.0.67 and hereafter, part of the setting are from 1.0.69 on.

1.0.67 supports

- Scan sound on/off switch

- Vibration on/off switch

- Automatically on when power on

- End char setting

- Barcode transmit method

- Barcode broadcast method

- Scanner activated/disabled setting

1.0.69 supports

- Continuous scan on/off switch

- Time interval setting

- Barcode prefix parameter setting

- Barcode suffix parameter setting

  Barcode parameters setting

1.1.3 supports

  Filer for the barcode leading and trailing space

  Non-printable character filter

Remark: Support multiple settings in one broadcast.

## Scan sound on/off switch

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | sound_play |
| Return to value | boolean type, true or false |

Example：

Intent intent = new Intent(“com.android.scanner.service_settings”);

intent.putExtra(“sound_play”, true);

sendBroadcast(intent);

## Vibration on/off switch

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | viberate |
| Return to value | boolean type, true or false |

Example：

Intent intent = new Intent(“com.android.scanner.service_settings”);

intent.putExtra(“viberate”, false);

sendBroadcast(intent);

## Continuous scan on/off switch

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | scan_continue |
| Return to value | boolean type, true or false |

Example：

Intent intent = new Intent("com.android.scanner.service_settings");

intent.putExtra("scan_continue", true);

sendBroadcast(intent);

## Time interval setting

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | interval |
| Return to value | Int type |

Example：

Intent intent = new Intent("com.android.scanner.service_settings");

intent.putExtra("interval", 3000);

sendBroadcast(intent);

## Barcode prefix parameter setting

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | prefix |
| Return to value | String type, default as " " |

Example：

Intent intent = new Intent("com.android.scanner.service_settings");

intent.putExtra("prefix", "");

sendBroadcast(intent);

## Barcode suffix parameter setting

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | suffix |
| Return to value | String type, default as " " |

Example：

Intent intent = new Intent("com.android.scanner.service_settings");

intent.putExtra("suffix", "");

sendBroadcast(intent);

## Automatically on when power on

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | boot_start |
| Return to value | boolean type, true or false |

Example：

Intent intent = new Intent(“com.android.scanner.service_settings”);

intent.putExtra(“boot_start”, true);

sendBroadcast(intent);

## End char setting

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | endchar |
| Return to value | Add end char behind the barcode, string types, please note the capitals, support the below listed strings: - “ENTER”: Enter - “TAB”: TAB - “SPACE”: Space - “NONE”: No end char |

Example：

Intent intent = new Intent(“com.android.scanner.service_settings”);

intent.putExtra(“endchar”, “ENTER”);

sendBroadcast(intent);

## Barcode transmit method

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | barcode_send_mode |
| Return to value | Transmitting method after barcode information well captured, string types, please note the capitals, support the below listed strings: “FOCUS”: Focus input “BROADCAST”: Broadcast “EMUKEY”: Simulate key “CLIPBOARD”: Clipboard |

Example：

Intent intent = new Intent(“com.android.scanner.service_settings”);

intent.putExtra(“barcode_send_mode”, “BROADCAST”);

sendBroadcast(intent);

## Barcode broadcast method

Barcode broadcast name

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | action_barcode_broadcast |
| Return to value | String type, default as: “com.android.server.scannerservice.broadcast” |

Barcode key value name

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | key_barcode_broadcast |
| Return to value | String type, default as: “scannerdata” |

Example：

Intent intent = new Intent(“com.android.scanner.service_settings”);

intent.putExtra(“action_barcode_broadcast”, 	“com.android.server.scannerservice.broadcast”);

intent.putExtra(“key_barcode_broadcast”, “scannerdata”);

sendBroadcast(intent);

## Filer for the barcode leading and trailing space

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | filter_prefix_suffix_blank |
| Return to value | boolean type, true or false |

Example：

Intent intent = new Intent("com.android.scanner.service_settings");

intent.putExtra("filter_prefix_suffix_blank", true);

sendBroadcast(intent);

## Barcode parameters setting

| action | com.seuic.scanner.action.PARAM_SETTINGS |
| --- | --- |
| extra1 | number |
| Return to value | int type |
| extra2 | value |
| Return to value | int type |

Barcode parameters info refer to : AndroidScan Interface document(1.0)

Example：

Intent intent = new Intent("com.seuic.scanner.action.PARAM_SETTINGS ");

intent.putExtra("number", 0x01);

intent.putExtra("value", 30);

sendBroadcast(intent);

## Scanner activated/disabled setting

| action | com.android.scanner.ENABLED |
| --- | --- |
| extra | enabled |
| Return to value | boolean type, true or false |

Example：

Activated the scanner：

Intent intent = new Intent("com.android.scanner.ENABLED");

intent.putExtra("enabled”, true);

sendBroadcast(intent);

Disabled the scanner：

Intent intent = new Intent("com.android.scanner.ENABLED");

intent.putExtra("enabled”, false);

sendBroadcast(intent);

## Non-printable character filter

| action | com.android.scanner.service_settings |
| --- | --- |
| extra | filter_invisible_chars |
| Return to value | boolean type, true or false |

Example：

Intent intent = new Intent("com.android.scanner.service_settings");

intent.putExtra("filter_invisible_chars ", true);

sendBroadcast(intent);

## Start Scan

| action | com.scan.onStartScan |
| --- | --- |

Example：

Start Scan：

Intent intent = new Intent("com.scan.onStartScan");

sendBroadcast(intent);

## Stop Scan

| action | com.scan.onEndScan |
| --- | --- |

Example：

Stop Scan：

Intent intent = new Intent("com.scan.onEndScan");

sendBroadcast(intent);

## Special Scan Function realization

## Continuous light mode setting

Example codes：

//Continuous scan on

Intent intent = new Intent("com.android.scanner.service_settings");

intent.putExtra("scan_continue", true);

sendBroadcast(intent);

//Set the barcode parameter continuous light mode on

Intent intentParam = new Intent("com.android.scanner.PARAM_SETTINGS ");

intent.putExtra("number", 0x0c);

intent.putExtra("value", 1);

sendBroadcast(intentParam);