P1

UhfReaderSdk Interface

Reader Connection

Connect to the Reader

| Method | synchronized Integer connectReader() |
| --- | --- |
| Parameter |  |
| Return value | Connection success or failure |
| Note |  |

Close the Reader

| Method | synchronized void closeReader() |
| --- | --- |
| Parameter |  |
| Return value | Connection success or failure |
| Note | Generally, this interface shall not be called, because other apps may be also using the reader |

Whether the Reader has been Opened

| Method | Boolean isReaderOpen() |
| --- | --- |
| Parameter |  |
| Return value | Opened or not |
| Note | Whether the reader module has been opened |

Get the Working State of Reader

| Method | Integer getReaderWorkState() |
| --- | --- |
| Parameter |  |
| Return value | Reader working state |
| Note |  |

- Working state of reader

P2

enum class ReaderWorkState(val state: Int) {

READER_CLOSE(0),//Close

READER_OPEN(1),//Open

READER_SEARCH(2),//Search card

READER_READ(3),//Read data

READER_WRITE(4),//Read data

READER_LOCKTAG(5),//Lock card

READER_DESTROYTAG(6),//Destroy card

}

Search Card

Register Card Search Listener

| Method | void registerReadListener(@Nullable IReadListener readListener) |
| --- | --- |
| Parameter | Listener |
| Return value |  |
| Note | Ensure that the AIDL service has been connected during registration |

- Listener

private static final IReadListener.Stub readListener = new IReadListener.Stub()

{

@Override

public void tagRead(List<TagInfo> tags) {

}

@Override

public void tagReadException(int errorCode) {

}

//AIDL service differentiation listener when used for registering the listener

@Override

public String getTag() throws RemoteException {

return "" + this.hashCode();

}

};

- TagInfo

// Which antenna reads the tag

private final byte AntennaID;

// From which frequency point is the tag read

private final int Frequency;

// Timestamp read by the tag, in milliseconds (relative to the moment the command is issued)

private final int TimeStamp;

// Addition al data length

private final short EmbeddedDataLen;

// Additional data

@Nullable

private final byte[] EmbeddedData;

P3

// epc length, in byte

private final short EpcLen;

// pc code, hexadecimal byte array

@NotNull

private final byte[] PC;

// pc code, hexadecimal byte array

@Nullable

private final byte[] EpcId;

// Tag protocol

@Nullable

private final String protocol;

// Signal strength

private final int RSSI;

- Register the listener again during AIDL connection

// Register the observer, and connect the reader and register the card search listener again after AIDL service connection

com.seuic.androidreader.sdk.Constants.INSTANCE.getConnectState().observeForever(

aidlInfoBean -> {

if (aidlInfoBean.getState() ==

com.seuic.androidreader.sdk.Constants.CONNECTED) {

WorkStateUtils.getINSTANCE().tryConnect();

TagListener.getINSTANCE().registerReadListener();

Log.e(TAG, "register the card search listener");

}

});

- AIDL Service connection state

const val DISCONNECT = 0 //Disconnected

const val CONNECTED = 1 //Connected

const val CONNECTING = 2 //Connecting

Cancel Card Search Listener

| Method | void unregisterReadListener(@Nullable IReadListener readListener) |
| --- | --- |
| Parameter | Listener |
| Return value |  |
| Note |  |

Clear Card Search Listener

| Method | void clearReadListeners() |
| --- | --- |
| Parameter |  |
| Return value |  |
| Note |  |

P4

Single Card Search

| Method | synchronized Integer inventoryOnce(@NotNull int[] ants, int timeout) |
| --- | --- |
| Parameter | ants: array of enabled antennas, timeout: timeout period (millisecond) |
| Return value | 0 success, non-0 failure |
| Note | The card search data is acquired through the listener |

Continuous Card Search

| Method | synchronized Integer inventoryStart(@NotNull int[] ants) |
| --- | --- |
| Parameter | ants: array of enabled antennas |
| Return value | 0 success, non-0 failure |
| Note | The card search data is acquired through the listener |

Stop Card Search

| Method | synchronized Integer inventoryStop() |
| --- | --- |
| Parameter |  |
| Return value | 0 success, non-0 failure |
| Note |  |

Tag Operation

Read the Tag

| Method | synchronized ParamsBackData<byte[]> readTagData(int antenna, int bank, int blockCount, int startAddress, @NotNull String accessPassword, @NotNull String epc) |
| --- | --- |
| Parameter | antenna: Antenna ports 1~8 bank: area (0, reserved area; 1, EPC area; 2, TID area; 3, user data area) blockCount: number of blocks (one character per block, and the characteris the unit) startAddress: Start address (in character) accessPassword: Access password epc: Tag EPC value to be filtered |
| Return value | Hexadecimal byte array |
| Note |  |

P5

Write the Tag

| Method | synchronized Integer writeTagData(int antenna, int bank, @NotNull String data, int startAddress, @NotNull String accessPassword, @NotNull String epc) |
| --- | --- |
| Parameter | antenna: Antenna ports 1~8 bank: area (0, reserved area; 1, EPC area; 2, TID area; 3, user data area) data: Hexadecimal character string startAddress: Start address (in character) accessPassword: Access password epc: Tag EPC value to be filtered |
| Return value | 0 success, non-0 failure |
| Note |  |

Lock the Tag

| Method | synchronized ParamsBackData<Integer> lockTag(int antenna, int lockBank, int lockType, @NotNull String accessPassword, @NotNull String epc) |
| --- | --- |
| Parameter | antenna: Antenna ports 1~8 lockBank: area (0, reserved area; 1, EPC area; 2, TID area; 3, user data area) lockType: Lock operation type (0, unlock; 1, lock; 2, permanent lock) accessPassword: Access password epc: Tag EPC value to be filtered |
| Return value | 0 success, non-0 failure |
| Note | The first two characters in the reserved area are the destruction password, and the last two characters are the access password; the access password must be changed before locking the card (it cannot be 00000000); when the reserved area is locked, the modified password needs to be used for both reading and writing; when locking other three areas, you may use 00000000 to read. However, if you need to write (the TID area is not writable), you must use the modified password |

Destroy the Tag

| Method | synchronized Integer killTag(int antenna, @NotNull String killPassword, @NotNull String epc) |
| --- | --- |
| Parameter | antenna: Antenna ports 1~8 killPassword: Destruction password epc: Tag EPC value to be filtered |
| Return value | 0 success, non-0 failure |
| Note |  |

P6

Reader Parameters

Get the Power

| Method | ParamsBackData<AntPowerData[]> getPower() |
| --- | --- |
| Parameter |  |
| Return value | Antenna array, power 1~33 |
| Note |  |

- AntPowerData

// Antenna id 1~8

var antid: Int = 0,

var readPower: Short = 0,

var writePower: Short = 0

Set the Power

| Method | Integer setPower(@NotNull AntPowerData[] antPowers) |
| --- | --- |
| Parameter | Antenna array, power 1~33 |
| Return value | 0 success, non-0 failure |
| Note |  |

Get the Frequency Band

| Method | ParamsBackData<String> getRegion() |
| --- | --- |
| Parameter | Frequency band character string (FCC, ETSI, China1, China2) |
| Return value | Frequency band |
| Note | FCC: North America, ETSI: Europe, China1: China1, China2: China2, KR: Korea |

Set the Frequency Band

| Method | Integer setRegion(String region) |
| --- | --- |
| Parameter | Frequency band character string |
| Return value | 0 success, non-0 failure |
| Note |  |

P7

Get the Embedded Data

| Method | ParamsBackData<EmbeddedBean> getEmbeddedData() |
| --- | --- |
| Parameter |  |
| Return value | Embedded data |
| Note |  |

- Embedded Bean

// Reserved area 0, TID area 2, USER area 3

private int bank;

// start address (in bytes)

private int startaddr;

// number of bytes

private int bytecnt;

// access password

@NotNull

private byte[] accesspwd;

// Whether to enable

private boolean enable;

Set the Embedded Data

| Method | Integer setEmbeddedData(int startAddress, int byteCount, int bank, String accessPassword, boolean enable) |
| --- | --- |
| Parameter | startAddress: Start address (in bytes) byteCount: Number of bytes bank: reserved area 0, TID area 2, USER area 3 access Password: Access password enable: Whether to enable additional data |
| Return value | 0 success, non-0 failure |
| Note | EPC area cannot be set |

Get the Filter

| Method | ParamsBackData<FilterBean> getFilter() |
| --- | --- |
| Parameter |  |
| Return value | Filter parameters |
| Note |  |

P8

- FilterBean

// Filter data areas 1, 2, 3, which are EPC, TID and User, respectively

private Integer bank;

// Filter data start address (bytes)

private Integer startAddress;

// Filter data

@NotNull

private String data;

//Match or not

private Boolean isInvert;

// Whether to enable filtering

private Boolean enable;

Set the Filter

| Method | Integer setFilter(int startAddress, int bank, String data, boolean isInvert, boolean enable) |
| --- | --- |
| Parameter | startAddress: start address (in bytes) bank: Filter data areas 1, 2, 3, which are EPC, TID and User, respectively data: Filtered data isInvert: Match or not enable: Whether to enable filtering |
| Return value | 0 success, non-0 failure |
| Note | After enable is set to true, it will always take effect, unless enable is set to false to clear the filter |

Get the Session

| Method | ParamsBackData<Integer> getSession() |
| --- | --- |
| Parameter |  |
| Return value | session |
| Note |  |

Set the Session

| Method | Integer setSession(@IntRange(from = 0L,to = 3L) int session) |
| --- | --- |
| Parameter | session 0~3 |
| Return value | 0 success, non-0 failure |
| Note |  |

P9

Get the Profile

| Method | ParamsBackData<Integer> getProfile() |
| --- | --- |
| Parameter |  |
| Return value | profile |
| Note |  |

Set the Profile

| Method | Integer setProfile(@IntRange(from = 1L,to = 4L)int profile) |
| --- | --- |
| Parameter | profile 1~4 |
| Return value | 0 success, non-0 failure |
| Note |  |

Get the Target

| Method | ParamsBackData<Integer> getTarget() |
| --- | --- |
| Parameter |  |
| Return value | target |
| Note |  |

Set the Target

| Method | Integer setTarget(@IntRange(from = 0L,to = 3L) int flag) |
| --- | --- |
| Parameter | target 0~3 |
| Return value | 0 success, non-0 failure |
| Note |  |

Get the qValue

| Method | ParamsBackData<Integer> getQValue() |
| --- | --- |
| Parameter |  |
| Return value | qValue |
| Note |  |

P10

Set the qValue

| Method | Integer setQValue(@IntRange(from = -1L,to = 15L) int qValue) |
| --- | --- |
| Parameter | q -1~15 |
| Return value | 0 success, non-0 failure |
| Note | q=-1 indicates automatic mode |

Get the Identified Antenna

| Method | ParamsBackData<InvantAntData> getInvantAnt() |
| --- | --- |
| Parameter |  |
| Return value | Identified antenna |
| Note |  |

- InvantAntData

// Total number of identified antennas

private int antcnt;

// Array of identified antennas

@NotNull

private int[] connectedants;

Get the Temperature

| Method | ParamsBackData<Integer> getTemperature() |
| --- | --- |
| Parameter |  |
| Return value | Temperature (Celsius) |
| Note |  |

Get the Reader Version

| Method | ParamsBackData<VersionData> getVersion() |
| --- | --- |
| Parameter |  |
| Return value | Reader version |
| Note |  |

- Version Data

P11

// Firmware information

@NotNull

private String firmdata;

// boot version

@NotNull

private String bootver;

// hardware version

@NotNull

private String hardver;

// Firmware version

@NotNull

private String firmver;

GPIO

Get the gpi

| Method | ParamsBackData<Integer> getGpi(int gpi) |
| --- | --- |
| Parameter | gpi: Port number 1~4 |
| Return value | 0 low level |
| Note |  |

Set the gpo

| Method | Integer setGpo(int gpo, int value) |
| --- | --- |
| Parameter | gpo: Ports 1~4 value: High and low level |
| Return value | 0 success, non-0 failure |
| Note |  |

LED Light

The Green Light Starts Flickering during Card Search

| Method | void startBling() |
| --- | --- |
| Parameter |  |
| Return value |  |
| Note | The green light will flicker when such interface is called during the continuous card search |

P12

The Green Light Stops Flickering during Card Search

| Method | void startBling() |
| --- | --- |
| Parameter |  |
| Return value |  |
| Note | The green light will be normally on when such interface is called during the continuous card search |

Set to Switch on or off the LED Light

| Method | void setLed(@IntRange(from = 0L,to = 3L) int port, boolean enable) |
| --- | --- |
| Parameter | Port: 0 red light, 1 green light, 2 blue light, 3 pink light enable:true On |
| Return value |  |
| Note | Generally such interface is not called |

Buzzer

Set the Buzzer Enabling

| Method | void setBZEnable(boolean enable) |
| --- | --- |
| Parameter | Whether to enable the buzzer |
| Return value |  |
| Note | It does not mean to make the buzzer sound, but means whether the buzzer can sound after the interface that sounds is called |

Get the Buzzer Enabling

| Method | Boolean getBZEnable() |
| --- | --- |
| Parameter |  |
| Return value | Whether the buzzer is available |
| Note | The interface of the buzzer that sounds can be still called when it is not available. However, it will not sound actually |

Buzzer Sounding

| Method | void startBZ() |
| --- | --- |
| Parameter |  |
| Return value |  |
| Note | The buzzer sounds for a long time, and it will stop automatically after more than 15 seconds |

P13

Buzzer Stop

| Method | void stopBZ() |
| --- | --- |
| Parameter |  |
| Return value |  |
| Note | Stop the buzzer from sounding |

Buzzer Starts Sounding during Card Search

| Method | void startBZForSearching() |
| --- | --- |
| Parameter |  |
| Return value |  |
| Note | When such interface is called during continuous card search, it will sound once a second |

Buzzer Stops Sounding during Card Search

| Method | void stopBZForSearching() |
| --- | --- |
| Parameter |  |
| Return value |  |
| Note | When such interface is called after the stop of continuous card search, the buzzer will stop sounding |

Encapsulation Type

- ParamsBackData

// Whether the operation is successful or not; 0 means success, and then fetch data after success

private int err;

// Acquired result

@Nullable

private Object data;

// Type of data

@Nullable

private Class classType;