| 东集 EUIC S |  |  |  |
| --- | --- | --- | --- |
| SLED UHF Interface User Manual |  |  |  |
| www.seuic.com |  |  |  |
| 修订记录 |  |  |  |
| 版本号 | 修订日期 | 修订内容 | 修订人员 |
| 1.0 | 2019/09/09 | 建立本文档 | 冯晓东 |
| 1.1 | 2019/09/11 | 优化接口名称定义 | 冯晓东 |

Content

Overview	4

Hardware Platform	4

Software Platform	4

Applicable Object	4

Calling Method	4

UHF Interfaces	4

UhfDevice Class	5

Instructions	5

Function Interface	6

1.	Start Bluetooth Search	6

2.	Cancel Bluetooth Search	6

3.	Bluetooth Connection	6

4.	Disconnect Bluetooth Collection	7

5.	Get Sled Name	7

6.	Get Sled SN	7

7.	Get Sled Version	7

8.	Set Sled Customized Name	7

9.	Get SledParameter	8

10.	Set SledParameter	8

11.	Get Battery Information	8

12.	Get Power	9

13.	Set Power	9

14.	Get Region	9

15.	Set Region	9

16.	Get Sled Temperature	9

17.	Single Seeking	10

18.	Start Continuous Seeking	10

19.	Stop Seeking	10

20.	Start Filtering Tags	10

21.	Read Tag	10

22.	Write Tag	11

23.	Block Write Tag	11

24.	Lock Tag	12

25.	Get Inventory Parameter	12

26.	Set Inventory Parameter	12

27.	Get Template Parameter	13

28.	Set Template Parameter	13

EPC Class	13

Class Member Variable	13

Instructions	13

Function Interface	13

1.	Get ID in EPC object	13

2.	Compare if IDs are the same	14

BatteryInfo Class	14

Class Member Variable	14

Instructions	14

InvParam Class	14

Class Member Variable	14

Instructions	15

TmplParam Class (Inherited from InvParam class)	15

Class Member Variable	15

Instructions	15

# Overview

This document describes the device-related interfaces provided on the SLED in order to help better use of the special features provided by the terminal.

## Hardware Platform

The SDK is applicable to the following terminal device:

- SLED

The use range is specified in the function interface description. If it is not specified, it is supported.

## Software Platform

The SDK is based on Android 5.1 and supports Android Studio development tools.

## Applicable Object

In addition to Android's standard features, developers who want to use UHF modules.

## Calling Method

- Add the jar package required by UHF in the project app/libsref directory, as shown below:

# UHF Interfaces

| Package Name | com.seuic.sleduhf |
| --- | --- |
| Package File | sleduhf-v1.0.0.jar |
| Included Classes | UhfDevice |
| Function | Provide UHF related control interfaces |

## UhfDevice Class

### Instructions

import com.seuic.uhf.UhfDevice;

UhfDevice mDevice = UhfDevice.getInstance(context);

UhfCallback Interface

| Package Name | com.seuic.sleduhf |
| --- | --- |
| Package File | sleduhf-v1.0.0.jar |
| Included Interface | UhfCallback |
| Function | Get searched Bluetooth device information |
| Function Interface | onGetBtInfo(BluetoothDevice btInfo) |
| Function | Get status of connected Bluetooth |
| Function Interface | onGetConnectStatus(int status) |
| Function | Get EPC callback interface for tag seeking |
| Function Interface | onGetTagsId(List<EPC> epcs) |
| Function | Get Sled Scan Button Status |
| Function Interface | onGetScanKeyMode(int mode) |

//Before using, register and implement the callback function

mDevice.registerUhfCallback(mUhfCallback)；

UhfCallback mUhfCallback = new UhfCallback(){

public void onGetBtInfo(BluetoothDevice btInfo){

//Get the searched Bluetooth name and MAC address

}

UhfCallback mUhfCallback = new UhfCallback(){

public void onGetConnectStatus(int status){

//0: Connected  -1: Disconnected  -2: Connection timed out

}

UhfCallback mUhfCallback = new UhfCallback(){

public void onGetTagsId(List<EPC> epcs){

//Get Tag EPC ID

}

UhfCallback mUhfCallback = new UhfCallback(){

public void onGetScanKeyMode(int mode){

//1: Start seeking  0: Stop seeking

}

}；

mDevice.unregisterUhfCallback(mUhfCallback)；

### Function Interface

- Start Bluetooth Search

boolean  startDiscovery(int timeout)

Parameter

timeout  Bluetooth search timeout

Return value

boolean; true: Success, false: Fail

Note: Get the Bluetooth information through the callback function onGetBTInfo (BluetoothDevice btInfo).

- Cancel Bluetooth Search

void  cancelDiscovery()

Parameter

Null

Return Value

Null

- Bluetooth Connection

void  connect(String address, int timeout)

Parameter

address

MAC address of connected Bluetooth

timeout

Bluetooth connection timeout

Return Value

Null

Note: Get the Bluetooth connection status through the callback function onGetConnectStatus (int status).

- Disconnect Bluetooth Collection

void  disconnect()

Parameter

Null

Return Value

Null

- Get Sled Name

String  getSledName()

Parameter

Flag

Null

Return Value

String; Not null: Success, null: Fail

- Get Sled SN

String  getSledSN()

Parameter

Flag

Null

Return Value

String; Not null: Success, null: Fail

- Get Sled Version

String  getSledVersion()

Parameter

Flag

Null

Return Value

String; Not null: Success, null: Fail (Sled Software Version & UHF Firmware Version)

- Set Sled Customized Name

boolean  setSledName(String name)

Parameter

name

Character name (Less than 20 characters)

Return Value

boolean; true: Success, false: Fail

- Get SledParameter

int  getSledParam(int param)

Parameter

Return Value

param /int

| Sled Parameter | Parameter Constant Name | Parameter Value |
| --- | --- | --- |
| Bluetooth sleep time when connected | PARAM_SLEEP_TIMEOUT | (Time: min) |
| Bluetooth sleep time when disconnected | PARAM_BTCONNECT_SLEEP_TIMEOUT | (Time: min) |
| Power off time | PARAM_POWEROFF_TIMEOUT | (Time: min) |
| Display Sled Name | PARAM_DISPLAY_NAME | 0=Use default name 1=Use custom name |
| Seek Sound Enable | PARAM_SOUND | 0=Disable 1=Enable |
| Scan Key Function | PARAM_SCANKEY_FUNCTION | 0=Continuous seeking 1=Single seeking 2=Specific seeking |
| Scan Key Seeking Mode | PARAM_SCANKEY_INVMODE | 0=Continue after releasing 1=Stop after releasing |
| Display PC Num | PARAM_DISPLAY_PC | 0=Do not display PC Num 1=Display PC Num |
| fastID Enable | PARAM_FASTID | 0=Disable fastID 1=Enable fastID |
| Import Template Num | PARAM_IMPORT_TMPLNUM | Template Num: 0~9 |

* Note: The import template number can only be set, not Get.

- Set SledParameter

boolean setSledParam(int param, int value)

Parameter

Return Value

As the table above

- Get Battery Information

BatteryInfo  getBatteryInfo()

Parameter

Null

Return Value

BatteryInfo Class Object

- Get Power

float  getPower ()

Parameter

Null

Return Value

int; Not 0: Success, 0: Fail (0~33 dBm)

- Set Power

boolean  setPower (float power)

Parameter

power

value (0~33 dBm)

Return Value

boolean; true: Success, false: Fail

- Get Region

int  getRegion ()

Parameter

Null

Return Value

String; Not Null: Success, null: Fail

Region Value: ”0: FCC”, ”1: China1”

- Set Region

boolean  setRegion (int region)

Parameter

region

Region Value: ”0: FCC”, ”1: China1”

Return Value

boolean; true: Success, false: Fail

- Get Sled Temperature

String getSledTemperature()

Parameter

Null

Return Value

String; Not null: Success, null: Fail (℃)

- Single Seeking

boolean  inventoryOnce(EPC epc, int timeout)

Parameter

epc

EPC Class object

timeout

Command timeout: ms

Return Value

boolean; true: Success, false: Fail

- Start Continuous Seeking

boolean  inventoryStart ()

Parameter

Null

Return Value

boolean; true:Success, false:Fail

Note: Get the EPC number of the tag through the callback function onGetTagsID (EPC epcs).

- Stop Seeking

boolean  inventoryStop ()

Parameter

Null

Return Value

boolean; true: Success, false: Fail

- Start Filtering Tags

boolean  inventorySelectStart(byte[] epc, int offset, int len)

Parameter

epc

Tag ID (EPC)

offset

EPC filter matching start address (in bytes)

len

EPC filter match length in bytes

Return Value

boolean; true: Success, false: Fail

Note: Get the EPC number of the tag through the callback function onGetTagsID (EPC epcs).

- Read Tag

boolean  readTagData (byte[] epc, byte[] password, int bank, int offset ,int len, byte[] data)

Parameter

epc

Tag ID (PC+EPC)

password

Access password (4 bytes in length)

bank

Tag storage area (0: password area 1: EPC area 2: TI area 3: user area)

offset

Start address (in bytes)

len

The length of the data to be read, in bytes

data

Store the read data

Return Value

boolean; true: Success, false: Fail

- Write Tag

boolean  writeTagData (byte[] epc, byte[] password, int bank, int offset ,int len, byte[] data)

Parameter

epc

Tag ID (PC+EPC)

password

Access password (4 bytes in length)

bank

Tag storage area (0: password area 1: EPC area 2: TI area 3: user area)

offset

Start address (in bytes)

len

The length of the data to be written, in bytes

data

Store the data to be written to the tag

Return Value

boolean; true: Success, false: Fail

- Block Write Tag

boolean  blockWriteTagData (byte[] epc, byte[] password, int bank, int offset ,int len, byte[] data)

Parameter

epc

Tag ID (PC+EPC)

password

Access password (4 bytes in length)

bank

Tag storage area (0: password area 1: EPC area 2: TI area 3: user area)

offset

Start address (in bytes)

len

The length of the data to be written, in bytes

data

Store the data to be block written to the tag

Return Value

boolean; true:Success, false:Fail

- Lock Tag

boolean lockTag(byte[] epc, byte[] password, int flag)

Parameter

epc

Tag ID (PC+EPC)

password

Access password (4 bytes in length)

flag

0,1,2,3 Unlock Tag (0: password area 1: EPC area 2: TI area 3: user area)

10,11,12,13锁标签 (10: password area 11: EPC area 12: TI area 13: user area)

Return Value

boolean; true:Success, false:Fail

- Get Inventory Parameter

InvParam getInvParam()

Parameter

Null

Return Value

InvParam Class object

- Set Inventory Parameter

boolean setInvParam(InvParam param)

Parameter

param

InvParam Class object

Return Value

boolean; true:Success, false:Fail

- Get Template Parameter

TmplParam getTmplParam(int num)

Parameter

num

Parameter Template Num (0~9)

Return Value

TmplParam Class object

- Set Template Parameter

boolean setTmplParam(int num, TmplParam param)

Parameter

num

Parameter Template Num (0~9)

param

TmplParam Class object

Return Value

boolean; true:Success, false:Fail

## EPC Class

### Class Member Variable

byte[] id;

int len;

int rssi;

int count;

### Instructions

import com.seuic.sleduhf.EPC;

EPC epc = new EPC();

### Function Interface

- Get ID in EPC object

String  getId()

Parameter

Null

Return Value

String; String in HEX

- Compare if IDs are the same

boolean equals(Object obj)

Parameter

EPC Class

Return Value

boolean; true: Success, false: Fail

## BatteryInfo Class

### Class Member Variable

int  level; //%

int  current;  //mA

int  voltage; //mV

float  temperature;  //℃

### Instructions

import com.seuic.sleduhf.BatteryInfo;

BatteryInfo batteryinfo = new BatteryInfo ();

## InvParam Class

### Class Member Variable

int profile;
int session;
int target;
int tagfocus;
int toggletarget;
int initQ;
int minQ;
int maxQ;
int retryCount;
int threshold;

### Instructions

import com.seuic.sleduhf.InvParam;

InvParam invparam = new InvParam();

## TmplParam Class (Inherited from InvParam class)

### Class Member Variable

String name;

### Instructions

import com.seuic.sleduhf.TmplParam;

TmplParam tmplparam = new TmplParam();