com_rfid_uhf_Device.DLL Dynamic Link Library Manual

V1.0

# Table of Contents

Table of Contents	2

1. Introduction	3

2. Application Integration	4

3. Programming Interface	5

3.1 Connect/Close the reader	5

3.1.1 OpenComPort [connect to the specified serial port] 	5

3.1.2 CloseSpecComPort [close the specified serial port] 	6

3.1.3 OpenNetPort [enable TCP connection] 	7

3.1.4 CloseNetPort [disable TCP connection] 	8

3.2. 18000-6C Standard command	9

3.2.1. Inventory_G2 [query command] 	9

3.2.2. InventoryMix_G2 [mixed query command] 	11

3.2.3. ReadData_G2 [command of reading data] 	13

3.2.4. WriteData_G2 [command of writing data] 	14

3.2.5. WriteEPC_G2 [command of broadcasting writing EPC number] 	15

3.2.6. KillTag _G2 [command of destroying tag] 	16

3.2.7. Lock_G2 [command of setting the read and write protection status of memory area] 	17

3.2.8. BlockErase_G2 [block erase command] 	19

3.3. Command of customizing reader	20

3.3.1. GetReaderInformation [command of getting reader information] 	20

3.3.2 SetAddress [command of setting reader address] 	21

3.3.3 SetInventoryScanTime [command of setting query command maximum response time] 	22

3.3.4. SetRfPower [command of setting power]	23

3.3.5. SetRegion [command of setting working frequency] 	24

3.3.6. SetBaudRate [command of setting baud rate] 	25

3.3.7. SetAntennaMultiplexing [command of configuring antenna] 	26

3.3.8. WriteRfPower [command of setting write power] 	27

3.3.9. ReadRfPower [command of reading the write power] 	28

3.3.10. RetryTimes [command of setting write retry times] 	29

3.3.11. SetReadMode [command of setting real-time mode] 	30

3.3.12. SetCheckAnt [command of antenna detection] 	31

3.3.13. GetSeriaNo [command of getting reader serial number] 	32

3.3.14. SetBeepNotification [command of setting buzzer] 	33

3.3.15. SetDRM [set DRM] 	34

3.3.16. GetDRM [read DRM] 	35

3.3.17. GetReaderTemperature [read temperature] 	36

3.3.18. MeasureReturnLoss [measure return loss] 	37

Appendix 1. Return Value Error Code Table	38

Appendix 2. ErrorCode Error Code Table	39

The development kit supports the development of java. Currently, it only supports windows operating system (32-bit and 64-bit).

The master computer application program operates the R2000 series 18000-6C format electronic tag reader through com_rfid_uhf_Device.DLL, and supports the master computer application program to open multiple readers on multiple communication ports at the same time.

R2000 series of electronic tag reader has UHFReader288MP, UHFReader288M, UHFReader82 and other models.

# 2. Application Integration

The development kit consists of the following files:

| File | Use |
| --- | --- |
| com_rfid_uhf_Device.dll | DLL dynamic link library, including all API interfaces |
| UHFREADER288MP.dll | DLL dynamic link library, called by com_rfid_uhf_Device.dll |
| Device.java | Java project source files |

| Programming Language | Description |
| --- | --- |
| Java | com_rfid_uhf_Device.dll, UHFREADER288MP.dll are copied to the java project directory, or directly copied to C:\WINDOWS\system32. |

# 3. Programming Interface

## 3.1 Connect/Close the reader

3.1.1 OpenComPort [connect to the specified serial port]

| Definition | public native int OpenComPort(int port,byte[]comAddr,byte baud,int[]PortHandle); |  |  |
| --- | --- | --- | --- |
| Description | Open the serial port connected to the reader. |  |  |
| Parameters | Name | Type | Remarks |
|  | Port | Int | Serial port number connected to the reader.1-COM1; 2-COM2… |
|  | ComAdr | byte[] | Enter the address of the reader, if the real address is unavailable, please enter the broadcast address 0xFF, after the function is called successfully, this parameter will return the real address. |
|  | Baud | byte | Serial communication baud rate. 0-9600; 1-19200; 2-38400; 5-57600; 6-115200. |
|  | PortHandle | Int[] | Return a handle to connect the reader, this handle is required for all API operations thereafter. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code | int Port = 1; //com1 byte[]comAddr=new byte[1]; comAddr[0]=(byte)255; byte baud=5;//57600bps int[] PortHandle= new int[1]; int result = OpenComPort(Port, comAddr, baud, PortHandle); if(result==0) { //Connection succeeded …… } |  |  |

3.1.2 CloseSpecComPort [close the specified serial port]

| Definition | public native int CloseSpecComPort(int PortHandle); |  |  |
| --- | --- | --- | --- |
| Description | Cancel the connection between the specified serial port and the reader and release the corresponding resources. |  |  |
| Parameters | Name | Type | Remarks |
|  | PortHandle | Int | The handle connected to the reader, the handle obtained by opening the serial port API. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code | No |  |  |

3.1.3 OpenNetPort [enable TCP connection]

| Definition | public native int OpenNetPort(int Port,String IPaddr,byte[]comAddr,int[] PortHandle); |  |  |
| --- | --- | --- | --- |
| Description | Enable the TCP connection to the reader. |  |  |
| Parameters | Name | Type | Remarks |
|  | Port | Int | The port number connected to the reader. |
|  | ipAddr | String | IP address of the reader. |
|  | ComAdr | byte[] | Enter the address of the reader, for TCP connection, the user can directly enter the broadcast address 255. |
|  | PortHandle | Int[] | Return a handle to connect the reader, this handle is required for all API operations thereafter. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code | int Port = 27011; String ipAddr=”192.168.0.250”; byte[]comAddr=new byte[1]; comAddr[0]=(byte)255; int[] PortHandle= new int[1]; int result = OpenNetPort(Port, ipAddr,comAddr, PortHandle); if(result==0) { //Connection succeeded …… } |  |  |

3.1.4 CloseNetPort [disable TCP connection]

| Definition | public native int CloseNetPort(int PortHandle); |  |  |
| --- | --- | --- | --- |
| Description | Disable the TCP connection to the reader and release the corresponding resources. |  |  |
| Parameters | Name | Type | Remarks |
|  | PortHandle | Int | The handle connected to the reader, the handle obtained by opening the TCP connection. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code | No |  |  |

## 3.2.18000-6C standard command

3.2.1. Inventory_G2 [query command]

| Definition | public native int Inventory_G2(byte[]comAddr,byte QValue,byte Session,byte MaskMem, byte[]MaskAdr,byte MaskLen, byte[]MaskData,byte MaskFlag,byte AdrTID,byte LenTID, byte TIDFlag,byte Target, byte InAnt,byte Scantime,byte FastFlag,byte[]pEPCList,byte[] Ant,int[]Totallen, int[]CardNum,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic tag that conforms to the protocol within the valid range. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | QValue | byte | [in] | 1 byte. bit7: Statistics data packet flag. 0 – After the query is over, do not send the query process statistics data packet will not be sent; 1 – After the query is over, send the query process statistics data packet will not be sent; Bit5: Fast ID query flag, only supports some Impinj Monza series tags, used to quickly obtain the EPC and TID of the tag. 0 - Disabled; 1 - Enabled. Bit4～bit0: The initial Q value used when querying the EPC tag, Note: 1. The setting of the Q value should be about the number of tags in the field equal to the Q power of 2. The Q value ranges 0～15. If other values appear in the command, the parameter error information will be returned. 2. The return query speed with statistical package is slower than that without. |
|  | Session | byte | [in] | 1 byte, the session value used when querying EPC tag. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. 0xff: The reader automatically configures the session (only valid for EPC query) It is recommended to select S0 for single or a small number of tag queries. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | byte[] | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | byte[] | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag bit.0 – not mask; 1 – mask. |
|  | AdrTID | byte | [in] | 1 byte, query the starting word address of TID area. |
|  | LenTID | byte | [in] | 1 byte, query the number of data words in TID area, range 0~15. |
|  | TidFlag | byte | [in] | 1 byte, TID query flag bit. 0 – Read EPC; 1 – Read TID. |
|  | Target | byte | [in] | 1 byte, query the Target value of tag. 0 – Target A; 1 – Target B. |
|  | InAnt | byte | [in] | 1 byte, the antenna number to be queried this time. 0x80 – Antenna 1; 0x81 – Antenna 2; 0x82 – Antenna 3; 0x83 – Antenna 4; The single-port reader is fixed at 0x80. |
|  | scantime | byte | [in] | 1 byte, the maximum time queried by this command, range 0-255 (*100ms). When the time is 0, it means there is no limit. |
|  | FastFlag | byte | [in] | 1 byte, flag bit of fast query. 0 - Disabled, no need to specify Target, Inant, Scantime has 3 parameters, the default is 0; 1 - Enabled, it needs to specify Target, Inant, Scantime 3 parameters. |
|  | pEPCList | byte[] | [out] | Returned tag data, format: EPC/TID length + EPC/TID number + RSSI. When there are multiple tags, several data in the same format are returned in order. |
|  | Ant | byte[] | [out] | 1 byte, query the antenna of the tag. 4-port reader, each bit represents the antenna number, such as 0x04, the binary is 0000 0100, which means that the antenna 3 queries the tag. 12-port reader, according to the numbers 0-11 correspond to antenna 1 to antenna 12 respectively. |
|  | Totallen | Int[] | [out] | Total length of data received by pEPCList. |
|  | CardNum | Int[] | [out] | Number of tags found in this query. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0x01: The command execution ends, and the queried electronic tag data is returned at the same time; 0x02: When the query time ends, the command execution is forced to exit, and the queried tag data is returned at the same time. 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected; For other values, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.2.2. InventoryMix_G2 [mixed query command]

| Definition | public native int InventoryMix_G2(byte[]comAddr,byte QValue,byte Session,byte MaskMem,byte[]MaskAdr,byte MaskLen,byte[]MaskData,byte MaskFlag,byte ReadMem, byte[]ReadAdr,byte ReadLen,byte[]Psd,byte Target,byte InAnt,byte Scantime,byte FastFlag,byte[]pEPCList,byte[] Ant,int[]Totallen,int[]CardNum,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic tag that conforms to the protocol within the valid range, and read the specified tag data while querying the EPC number of tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | QValue | byte | [in] | 1 byte. bit7: Statistics data packet flag. 0 – After the query is over, do not send the query process statistics data packet will not be sent; 1 – After the query is over, send the query process statistics data packet will not be sent; bit6~bit0: The initial Q value used when querying EPC tag, Note: 1. The setting of the Q value should be the number of tags in the field approximately equal to the Q power of 2. The Q value ranges 0～15. If other values appear in the command, the parameter error information will be returned. 2. The sending speed of data with statistical packet is slower than that without. |
|  | Session | byte | [in] | 1 byte, the session value used when querying the tag. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. It is recommended to select S0 for single or a small number of tag queries. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | byte[] | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | byte[] | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag bit.0 – not mask; 1 – mask. |
|  | ReadMem | byte | [in] | 1 byte, select the memory area to read, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | ReadAdr | byte[] | [in] | 2 bytes, the starting word address of the memory data to be read, the high byte first. |
|  | ReadLen | byte | [in] | 1 byte, the word length of the memory data to be read. |
|  | Psd | byte[] | [in] | 4 bytes, access password of the tag. |
|  | Target | byte | [in] | 1 byte, query the Target value of tag. 0 – Target A; 1 – Target B. |
|  | InAnt | byte | [in] | 1 byte, the antenna number to be queried this time. 0x80 – Antenna 1; 0x81 – Antenna 2; 0x82 – Antenna 3; 0x83 – Antenna 4; The single-port reader is fixed at 0x80. |
|  | scantime | byte | [in] | 1 byte, the maximum time queried by this command, range 0-255 (*100ms). When the time is 0, it means there is no limit. |
|  | FastFlag | byte | [in] | 1 byte, flag bit of fast query. 0 - Disabled, no need to specify Target, Inant, Scantime has 3 parameters, the default is 0; 1 - Enabled, it needs to specify Target, Inant, Scantime 3 parameters. |
|  | pEPCList | byte[] | [out] | Returned tag data, format: Rolling code + EPC/data length + EPC/data + RSSI. When there are multiple tags, several data in the same format are returned in order. The highest digit of the rolling code is 0, which means the current EPC, and the highest digit is 1, which means the memory data. The same tag data is uploaded with the EPC number first, and then the memory data. If the lower 7 bits are adjacent, it can be confirmed that it is the same data, if they are not adjacent, it is not the same tag data, the lower 7 digits of the rolling code are from 0 to 127, and the serial number is incremented by 1 each time it is uploaded. If it exceeds 127, it will start over from 0 |
|  | Ant | byte[] | [out] | 1 byte, query the antenna of the tag. 4-port reader, each bit represents the antenna number, such as 0x04, the binary is 0000 0100, which means that the antenna 3 queries the tag. 12-port reader, according to the numbers 0-11 correspond to antenna 1 to antenna 12 respectively. |
|  | Totallen | Int[] | [out] | Total length of data received by pEPCList. |
|  | CardNum | Int[] | [out] | Number of tags found in this query. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0x01: The command execution ends, and the queried electronic tag data is returned at the same time; 0x02: When the query time ends, the command execution is forced to exit, and the queried tag data is returned at the same time. 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected; For other values, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.3.ReadData_G2 [command of reading data]

| Definition | public native int ReadData_G2(byte[]comAddr,byte[] EPC,byte ENum,byte Mem,byte WordPtr,byte Num,byte[]Password, byte MaskMem,byte[]MaskAdr,byte MaskLen,byte[] MaskData,byte[]Data,int[]Errorcode, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command reads the data in all or part of the reserved area of the tag, EPC memory, TID memory or user memory.Start reading from the specified address, in word units. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | byte[] | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to read, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | byte | [in] | 1 byte, starting word address to read. |
|  | Num | byte | [in] | 1 byte, the word length to be read. |
|  | Password | byte[] | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | byte[] | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | byte[] | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | Data | byte[] | [out] | Num words, the read tag data. |
|  | ErrorCode | Int[] | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.4. WriteData_G2 [command of writing data]

| Definition | public native int WriteData_G2(byte[]comAddr,byte[] EPC,byte WNum,byte ENum,byte Mem,byte WordPtr,byte[]Writedata,byte[]Password,byte MaskMem,byte[]MaskAdr,byte MaskLen,byte[]MaskData,int[]Errorcode,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command can write several words to reserved memory, EPC memory, TID memory or user memory at one time. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | byte[] | [in] | ENum words, the EPC number of tag. |
|  | Wnum | byte | [in] | 1 byte, the number of words to be written, when Enum=255, the mask method is used. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag. When Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to be written, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | byte | [in] | 1 byte, the starting word address to be written. |
|  | Writedata | byte[] | [in] | WNum bytes, the data to be written. |
|  | Password | byte[] | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | byte[] | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | byte[] | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int[] | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.5. WriteEPC_G2 [command of broadcasting writing EPC number]

| Definition | public native int WriteEPC_G2(byte[]ComAddr,byte[] Password,byte[] EPC,byte ENum,int[]Errorcode,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command writes the EPC number to the electronic tag. When writing, there can only be one electronic tag within the effective range of the antenna. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | Password | byte[] | [in] | 4 bytes, access password of the tag. |
|  | EPC | byte[] | [in] | Enum bytes, the EPC number to be written. |
|  | ENum | byte | [in] | 1 byte, the length of the tag word to be written. |
|  | ErrorCode | Int[] | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.6. KillTag _G2 [command of destroying tag]

| Definition | public native int KillTag_G2(byte[]ComAddr,byte[] EPC,byte ENum,byte[] Killpwd,byte MaskMem,byte[]MaskAdr,byte MaskLen,byte[]MaskData,int[]Errorcode,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to destroy the tag.Once the tag is destroyed, the commands of the memory will never be processed again. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | byte[] | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Killpwd | byte[] | [in] | 4 bytes, the kill password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | byte[] | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | byte[] | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int[] | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.7. Lock_G2 [command of setting read and write protection status of memory area]

| Definition | public native int Lock_G2(byte[]comAddr,byte[] EPC,byte ENum,byte selectid,byte setprotect,byte[] Password,byte MaskMem, byte[]MaskAdr,byte MaskLen,byte[]MaskData, int[]Errorcode,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command can set the reserved area to be readable and writable, always readable and writable, readable and writable with password, and always readable and writable, and can set EPC memory, TID memory and user memory as writable, always writable, writable with password, never writable. EPC memory, TID memory or user memory is always readable. Moreover, TID memory is read-only and can never be written. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | byte[] | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | selectid | byte | [in] | 1 byte, 0x00 - Control the Kill password read and write protection settings. 0x01 – Control the access password read and write protection settings. 0x02 - Control the EPC memory read and write protection settings. 0x03 - Control the TID memory read and write protection settings. 0x04 - Control the user memory read and write protection settings. Other values are reserved. If the reader receives other values, it will return a message of parameter error. |
|  | setprotect | byte | [in] | 1 byte, When Select is 0x00 or 0x01, the meaning of the SetProtect value is as follows: 0x00 - Set to readable and writable 0x01 - Set to always readable and writable 0x02 - Set to readable and writable with password 0x03 - Set to never readable and writable When Select is 0x02, 0x03, 0x04, the meaning of the SetProtect value is as follows: 0x00 - Set to writable 0x01 - Set to always writable 0x02 - Set to writable with password 0x03 - Set to never writable. |
|  | Password | byte[] | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | byte[] | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | byte[] | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int[] | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.8. BlockErase_G2 [block erase command]

| Definition | public native int BlockErase_G2(byte[]ComAddr,byte[] EPC,byte ENum,byte Mem,byte WordPtr,byte Num,byte[] Password,byte MaskMem,byte[]MaskAdr,byte MaskLen,byte[]MaskData,int[]Errorcode,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command erases several words in the reserved memory, EPC memory, TID memory, or user memory of the tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | byte[] | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to be erased, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | byte[] | [in] | 1 byte, starting word address to be erased. |
|  | Num | byte | [in] | 1 byte, word length to be erased. |
|  | Password | byte[] | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | byte[] | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | byte[] | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int[] | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

## 3.3. Command of customizing reader

3.3.1. GetReaderInformation [command of getting reader information]

| Definition | public native int GetReaderInformation(byte[]comAddr,byte[]versionInfo,byte[]readerType, byte[]trType,byte[]dmaxfre,byte[]dminfre,byte[]powerdBm,byte[]ScanTime,byte[]Ant,byte[]BeepEn, byte[]OutputRep,byte[]CheckAnt,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader information, including reader address (ComAdr) and software version (VersionInfo) etc. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | 1 byte, use the reader address returned by the connected reader. |
|  | VersionInfo | byte[] | [out] | 2 bytes, reader version information. The first byte is the version number, and the second byte is the handle of the subversion number. |
|  | ReaderType | byte[] | [out] | 1 byte, type code of the reader. |
|  | dmaxfre | byte[] | [out] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current maximum frequency of the reader. |
|  | dminfre | byte[] | [out] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current minimum frequency of the reader. |
|  | PowerDbm | byte[] | [out] | 1 byte, the output power of the reader. The range is 0-30 in dBm. |
|  | scantime | byte[] | [out] | 1 byte, query time. After the reader receives the query command, it will reply to the master computer within the query time. |
|  | Ant | byte[] | [out] | 1 byte, antenna configuration information. |
|  | BeepEn | byte[] | [out] | 1 byte, buzzer switch information. |
|  | OutputRep | byte[] | [out] | 1 byte, notice output port parameters. |
|  | CheckAnt | byte[] | [out] | 1 byte, antenna detection information. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

Note: The frequency band settings are as follows:

| MaxFre(Bit7) | MaxFre(Bit6) | MinFre(Bit7) | MinFre(Bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

3.3.2 SetAddress [command of setting reader address]

| Definition | public native int SetAddress(byte[]comAddr, byte ComAdrData,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | After executing this command, the reader will change the address of the reader to the value given by the user, and write and save this value into the EEPROM. The factory default value is 0x00. The modification range allowed by the user is 0x00~0xFE. When the value written by the user is 0xFF, the reader will automatically return to the default value of 0x00. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | ComAdrData | byte | [in] | 1 byte, the new address after modification. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.3 SetInventoryScanTime [command of setting query command maximum response time]

| Definition | public native int SetInventoryScanTime(byte[]comAddr, byte ScanTime,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the maximum response time of the query command. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | scantime | byte | [in] | 1 byte, query command response time, range 0-255. When set to 0, this parameter is invalid and waits until the round is read. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.4. SetRfPower [command of setting power]

| Definition | public native int SetRfPower(byte[]comAddr, byte PowerDbm,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the power of reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | PowerDbm | byte | [in] | 1 byte, reader power, range 0~30dBm. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.5. SetRegion [command of setting working frequency]

| Definition | public native int SetRegion(byte[]comAddr, byte dmaxfre,byte dminfre,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command sets the upper limit frequency and lower limit frequency of the reader. The upper limit frequency must be greater than or equal to the lower limit frequency. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | dmaxfre | byte | [in] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current maximum frequency of the reader. |
|  | dminfre | byte | [in] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current minimum frequency of the reader. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

Note: The frequency band settings are as follows:

| MaxFre(Bit7) | MaxFre(Bit6) | MinFre(Bit7) | MinFre(Bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band |

The calculation formula of each frequency band:

Chinese band2: Fs = 920.125 + N * 0.25 (MHz), where N∈[0, 19].

US band: Fs = 902.75 + N * 0.5 (MHz), where N∈[0,49].

Korean band: Fs = 917.1 + N * 0.2 (MHz), where N∈[0, 31].

EU band:Fs = 865.1 + N*0.2(MHz), where N∈[0, 14].

Ukraine band:Fs = 868.0 + N*0.1(MHz), where N∈[0, 6].

Chinese band1: Fs = 840.125 + N * 0.25 (MHz), where N∈[0, 19].

US band3: Fs = 902 + N * 0.5 (MHz), where N∈[0,52].

3.3.6. SetBaudRate [command of setting baud rate]

| Definition | public native int SetBaudRate(byte[]comAddr, byte baud,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the communication baud rate of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | Baud | byte | [in] | 1 byte, baud rate. 0 – 9600bps; 1 – 19200bps; 2 – 38400bps; 5 – 57600bps; 6 – 115200bps. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.7. SetAntennaMultiplexing [command of configuring antenna]

| Definition | public native int SetAntennaMultiplexing(byte[]comAddr, byte Ant,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used for 4-port reader antenna configuration. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | Ant | byte | [in] | 1 byte, antenna configuration information. At least one antenna must be configured. Bit0: Antenna 1 configuration bit. Bit0=0 Antenna 1 is off; Bit0=1 Antenna 1 is on. Bit1: Antenna 2 configuration bit. Bit1=0 Antenna 2 is off; Bit1=1 Antenna 2 is on. Bit2: Antenna 3 configuration bit. Bit2=0 Antenna 3 is off; Bit2=1 Antenna 3 is on. Bit3: Antenna 4 configuration bit. Bit3=0 Antenna 4 is off; Bit3=1 Antenna 4 is on. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.8. WriteRfPower [command of setting write power]

| Definition | public native int WriteRfPower(byte[]comAddr, byte PowerDbm,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the power when writing tags. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | PowerDbm | byte | [in] | 1 byte, the write power of the reader, 0-30; bit7=0 - Disable; bit7=1 – Enable. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.9. ReadRfPower [command of reading the write power]

| Definition | public native int ReadRfPower(byte[]comAddr, byte[] PowerDbm,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read the power when writing the tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | PowerDbm | byte[] | [out] | 1 byte, the write power of the reader, 0-30; bit7=0 - Disable; bit7=1 – Enable. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.10. RetryTimes [command of setting write retry times]

| Definition | public native int RetryTimes(byte[]comAddr, byte[] Times,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read the power when writing the tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | Times | byte[] | [out] | 1 byte, bit7=1 - Set the retry times; bit7=0 – Read the retry times; range 0-7. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.11. SetReadMode [command of setting real-time mode]

| Definition | public native int SetReadMode(byte[]comAddr, byte ReadMode,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the real-time query mode of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte | [in/out] | Use the reader address returned by the connected reader. |
|  | ReadMode | byte | [in] | 1 byte, 0 – Answer mode; 1 – Real-time query mode; 2 – Trigger mode. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.12. SetCheckAnt [command of antenna detection]

| Definition | public native int SetCheckAnt(byte[]comAddr, byte CheckAnt,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set whether to perform antenna detection during the process of reading and writing tags. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | CheckAnt | byte | [in] | 1 byte, antenna detection switch. 0x00 - Disable antenna detection during tag reading and writing. 0x01 – Enable antenna detection during tag reading and writing. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.13. GetSeriaNo [command of getting reader serial number]

| Definition | public native int GetSeriaNo(byte[]comAddr, byte[] SeriaNo,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to get the 4-byte unique serial number of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | SeriaNo | byte[] | [out] | 4 bytes, serial number of the reader. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.14. SetBeepNotification [command of setting buzzer]

| Definition | public native int SetBeepNotification(byte[]comAddr, byte BeepEn,int PortHandle);; |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the buzzer switch. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | BeepEn | byte | [in] | 1 byte, 0x00 - buzzer off; 0x01 - Buzzer on. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.15. SetDRM [set DRM]

| Definition | public native int SetDRM(byte[] ComAddr, byte DRM, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the DRM state. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | DRM | byte | [in] | 1 byte, DRM status, 0 - Off; 1 – On. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.16. GetDRM [read DRM]

| Definition | public native int GetDRM(byte[] ComAddr, byte[] DRM, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read DRM status. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | DRM | byte[] | [out] | 1 byte, DRM status, 0 - Off; 1 – On. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.17. GetReaderTemperature [read temperature]

| Definition | public native int GetReaderTemperature (byte[]ComAddr, byte[] PlusMinus,byte[] Temperature, int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read the current temperature of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | PlusMinus | byte[] | [out] | 1 byte, temperature status, 0 – Minus zero; 1 – Above zero. |
|  | Temperature | byte[] | [out] | 1 byte, temperature value, unit °c |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.3.18. MeasureReturnLoss [measure return loss]

| Definition | public native int MeasureReturnLoss(byte[]ComAddr, byte[] TestFreq, byte Ant, byte[] ReturnLoss,int PortHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to measure the return loss of the specified port of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | byte[] | [in/out] | Use the reader address returned by the connected reader. |
|  | TestFreq | byte[] | [in] | 4 bytes, measurement frequency. |
|  | Ant | byte | [in] | 1 byte, measurement antenna port, 0 – Antenna 1; 1 – Antenna 2. 2 – Antenna 3; 3 – Antenna 4. |
|  | ReturnLoss | byte[] | [out] | 1 byte, return loss, in dB. |
|  | PortHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

# Appendix 1.Return Value Error Code Table

| Error Code | Description |
| --- | --- |
| 0x00 | No error, indicating API call successful. |
| 0x01 | Return before the query time expires. |
| 0x02 | The specified query time overflow. |
| 0x05 | Access password error. |
| 0x09 | Destroy password error. |
| 0x0A | Destroy password shall not be all 0s. |
| 0x0B | Electronic tag does not support this command. |
| 0x0C | For this command, the access password shall not be 0. |
| 0x0D | The electronic tag has been set with reading protection and cannot be set again. |
| 0x0E | The electronic tag is not set with reading protection and does not need to be unlocked. |
| 0x10 | Byte space locked and writing failed. |
| 0x11 | Can not be locked. |
| 0x12 | Already locked and cannot be locked again. |
| 0x13 | The parameter saving failed, but the set value is valid until the read-write module is powered off. |
| 0x14 | Unable to adjust. |
| 0x15 | Return before the query time expires. |
| 0x17 | There is message after this message. |
| 0x18 | The storage space of the read-write module is full. |
| 0x19 | Electron does not support this command or the access password shall not be 0 |
| 0xF8 | Antenna detection error. |
| 0xF9 | Command execution error. |
| 0xFA | There is an electronic tag, but the communication is poor and cannot be operated. |
| 0xFB | No electronic tag is available for operating. |
| 0xFC | Electronic tag returns an error code. |
| 0xFD | Command length error. |
| 0xFE | Illegal order. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |
| 0x33 | Communication is busy and the device is executing other commands. |
| 0x35 | Port is open |
| 0x37 | Invalid handle. |

# Appendix 2.ErrorCode Error Code Table

| Error Code | Description |
| --- | --- |
| 0x00 | Other errors, all catch errors not covered by other codes. |
| 0x03 | Memory overrun or unsupported PC value, the specified storage location does not exist or PC value not supported by the tag. |
| 0x04 | Memory lock, memory location lock is permanently locked and cannot be written. |
| 0x0B | Insufficient power supply, insufficient tag power to perform storage and writing operations. |
| 0x0F | Non-specific error, the tag does not support specific error code. |