UHFREADER288MP.DLL Dynamic Link Library Manual

V2.0

# Table of Contents

Table of Contents	2

1. Overview	4

2. Application Integration	5

3. Programming Interface	6

3.1 Connect/Close the reader	6

3.1.1 AutoOpenComPort [automatically connect to serial port] 	6

3.1.2 OpenComPort [connect to specified serial port] 	7

3.1.3 CloseComPort [close all serial ports] 	8

3.1.4 CloseSpecComPort [close the specified serial port] 	9

3.1.5 OpenNetPort [enable TCP connection] 	10

3.1.6 CloseNetPort [disable TCP connection] 	11

3.2. 18000-6C standard command	12

3.2.1. Inventory_G2 [query command] 	12

3.2.2. InventoryMix_G2 [mixed query command] 	14

3.2.3. InitRFIDCallBack [initialization callback] 	17

3.2.4. ReadData_G2 [command of read data] 	18

3.2.5. WriteData_G2 [command of write data] 	19

3.2.6. BlockWrite_G2 [command of block write] 	20

3.2.7. ExtReadData_G2 [command of extended read data] 	21

3.2.8. ExtWriteData_G2 [command of extended write data] 	22

3.2.9. WriteEPC_G2 [command of broadcasting write EPC number] 	23

3.2.10. BlockErase_G2 [block erase command] 	24

3.2.11. Lock_G2 [command of setting read and write protection status of the memory area] 	25

3.2.12. KillTag _G2 [command of destroying tag] 	27

3.2.13. SingleTagInventory_G2 [single tag query command] 	28

3.3. Cache operation command	29

3.3.1. InventoryBuffer_G2 [cache query command] 	29

3.3.2. ReadBuffer_G2 [command of getting cache data] 	31

3.3.3. ClearBuffer_G2 [command of clearing cache data] 	32

3.3.4. GetBufferCnt_G2 [command of getting the number of cached tags] 	33

3.3.5. SetSaveLen [command of setting the EPC/TID length of cache] 	34

3.3.5. GetSaveLen [command of getting the length of cached EPC/TID] 	35

3.4. NXP tag command	36

3.4.1. SetPrivacyByEPC _G2 [command of setting single read protection] 	36

3.4.2. SetPrivacyWithoutEPC_G2 [command of setting read protection of multiple tags] 	37

3.4.3. ResetPrivacy_G2 [command of unlocking read protection] 	38

3.4.4. CheckPrivacy_G2 [command of detecting read protection] 	39

3.4.5. EASConfigure_G2 [command of configuring EAS alarm] 	40

3.4.6. EASAlarm_G2 [command of detecting alarm] 	41

3.5. Impinj Monza4 QT tag command	42

3.5.1. GetMonza4QTWorkParamter_G2 [command of read Monza4QT working parameters] 	42

3.5.2. SetMonza4QTWorkParamter_G2 [command of setting Monza4QT working parameters] 	43

3.5.3. Inventory_QT_G2 [QT query command] 	43

3.5.4. SetTagCustomFunction_G2 [command of customizing tag] 	46

3.6. 18000-6B standard command	47

3.6.1. InventorySingle_6B [single tag query command] 	47

3.6.2. InventoryMultiple_6B [conditional query command] 	48

3.6.3. ReadData_6B [command of read data] 	49

3.6.4. WriteData_6B [command of write data] 	50

3.6.5. CheckLock_6B [command of detecting lock] 	51

3.6.6. Lock_6B [command of locking] 	52

3.7. Command of customizing reader	53

3.7.1. GetReaderInformation [command of getting reader information] 	53

3.7.2 SetAddress [command of setting reader address] 	55

3.7.3 SetInventoryScanTime [command of setting the maximum response time of query command] 	56

3.7.4. SetRfPower [command of setting power] 	57

3.7.5. SetRegion [command of setting working frequency] 	58

3.7.6. SetBaudRate [command of setting baud rate] 	59

3.7.7. BuzzerAndLEDControl [acousto-optic control command] 	60

3.7.8. SetAntennaMultiplexing [command of configuring antenna] 	61

3.7.9. SetBeepNotification [command of setting buzzer] 	62

3.7.10. SetRelay [command of controlling relay] 	63

3.7.11. GetGPIOStatus [command of read GPIO statu] 	64

3.7.12. SetGPIO [command of controlling GPIO] 	65

3.7.13. GetSeriaNo [command of getting reader serial number] 	66

3.7.14. SetCheckAnt [command of detecting antenna] 	67

3.7.15. WriteRfPower [command of setting write power] 	68

3.7.16. ReadRfPower [command of read the write power] 	69

3.7.17. RetryTimes [command of setting write retry times] 	70

3.7.18. SetReadMode [command of setting real-time mode] 	71

3.7.19. SetReadParameter [command of setting working parameters in real-time mode] 	72

3.7.20. GetReadParameter [command of read working parameters in real-time mode] 	74

3.7.21. SetDRM [set DRM] 	75

3.7.22. GetDRM [read DRM] 	76

3.7.23. GetReaderTemperature [read temperature] 	77

3.7.24. MeasureReturnLoss [measure return loss] 	78

3.7.25. SetNotificationPulseOutput [command of notice output port] 	79

3.7.26. ReadActiveModeData [get active uploaded data] 	80

Appendix 1.Return Value Error Code Table	81

# Appendix 2.ErrorCode Error Code Table	82

# 1. Overview

The development kit supports C, C + + and other languages that can call the C function interface. Such as delphi, C#, VB6.0, VB.NET and so on. The development kit currently only supports windows operating system (32-bit and 64-bit).

The master computer application program operates the R2000 series 18000-6C, 18000-6B format electronic tag reader through UHFREADER288MP.DLL, and supports the master computer application program to open multiple readers on multiple communication ports at the same time.

R2000 series of electronic tag reader has UHFReader288MP, UHFReader288M, UHFReader82 and other models.

# 2. Application Integration

The development kit consists of the following files:

| File | Use |
| --- | --- |
| UHFREADER288MP.dll | DLL dynamic link library, including all API interfaces |
| UHFREADER288MP.lib | VC + + project static link library |
| UHFREADER288MP.h | C/C + + project header file, function definition |
| Reader.h | C/C + + additional header file |
| ThreadClass.h | C/C + + additional header file |
| RWDev.cs | C# function definition class file |
| UHFREADER288MP_DLL_Head.pas | Delphi function definition header file |

| Programming Language | Description |
| --- | --- |
| VC + + | Copy UHFREADER288MP.lib, UHFREADER288MP.h, Reader.h, ThreadClass.h to the project directory and add to the project. Copy UHFREADER288MP.dll to the exe file output directory. |
| C# | Copy RWDev.cs to the project directory and add it to the project. Copy UHFREADER288MP.dll to the exe file output directory. |
| Delphi | Copy UHFREADER288MP_DLL_Head.pas to the project directory and add it to the project. Copy UHFREADER288MP.dll to the exe file output directory. |

# 3. Programming Interface

## 3.1 Connect/Close the reader

3.1.1 AutoOpenComPort [automatically connect to serial port]

| Definition | int AutoOpenComPort(int* port, BYTE* ComAdr, BYTE baud, int* FrmHandle); |  |  |
| --- | --- | --- | --- |
| Description | Automatically identify and open the serial port connected to the reader, this function only supports the serial port between com1-com9. |  |  |
| Parameters | Name | Type | Remarks |
|  | Port | Int* | Return the serial port number connected to the reader. |
|  | ComAdr | BYTE* | Enter the address of the reader, if the real address is unavailable, please enter the broadcast address 0xFF, after the function is called successfully, this parameter will return the real address. |
|  | Baud | byte | Serial communication baud rate.0-9600; 1-19200; 2-38400; 5-57600; 6-115200. |
|  | FrmHandle | Int* | Return a handle to connect the reader, this handle is required for all API operations thereafter. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code | int port=0; BYTE address=255;//Broadcast address BYTE baud=5;//57600 int FrmHandle=-1; int fCmdRet =AutoOpenComPort(&port,&address, baud, &FrmHandle); |  |  |

3.1.2 OpenComPort [connect to specified serial port]

| Definition | int OpenComPort(int port, BYTE* ComAdr, BYTE baud, int* FrmHandle) ; |  |  |
| --- | --- | --- | --- |
| Description | Open the serial port connected to the reader. |  |  |
| Parameters | Name | Type | Remarks |
|  | Port | Int | Serial port number connected to the reader.1-COM1;2-COM2… |
|  | ComAdr | BYTE* | Enter the address of the reader, if the real address is unavailable, please enter the broadcast address 0xFF, after the function is called successfully, this parameter will return the real address. |
|  | Baud | byte | Serial communication baud rate.0-9600; 1-19200; 2-38400; 5-57600; 6-115200. |
|  | FrmHandle | Int* | Return a handle to connect the reader, this handle is required for all API operations thereafter. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code | int port=1;//Open serial port com1 BYTE address=255;//Broadcast address BYTE baud=5;//57600 int FrmHandle=-1; int fCmdRet =OpenComPort(port,&address, baud, &FrmHandle); if(fCmdRet==0) { //Connection succeeded ........... } |  |  |

3.1.3 CloseComPort [close all serial ports]

| Definition | int CloseComPort(); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Cancel all serial port and reader connections and release corresponding resources. |  |  |  |
| Parameters (None) | Name | Type | Direction | Remarks |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |  |
| Reference code |  |  |  |  |

3.1.4 CloseSpecComPort [close the specified serial port]

| Definition | int CloseSpecComPort(int FrmHandle); |  |  |
| --- | --- | --- | --- |
| Description | Cancel the connection between the specified serial port and the reader and release the corresponding resources. |  |  |
| Parameters | Name | Type | Remarks |
|  | FrmHandle | Int | The handle connected to the reader, the handle obtained by opening the serial port API. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code |  |  |  |

3.1.5 OpenNetPort [enable TCP connection]

| Definition | Int OpenNetPort(int Port, LPCTSTR IPaddr, BYTE*ComAdr, int *Frmhandle); |  |  |
| --- | --- | --- | --- |
| Description | Enable the TCP connection to the reader. |  |  |
| Parameters | Name | Type | Remarks |
|  | Port | Int | The port number connected to the reader. |
|  | ipAddr | LPCTSTR | IP address of the reader. |
|  | ComAdr | BYTE* | Enter the address of the reader, for TCP connection, the user can directly enter the broadcast address 255. |
|  | FrmHandle | Int* | Return a handle to connect the reader, this handle is required for all API operations thereafter. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code | int port=6000; LPCTSTR ipAddr = "192.168.1.190"; BYTE ComAdr =255;//Broadcast address int FrmHandle=-1; int fCmdRet =OpenNetPort(port, ipAddr ,&ComAdr, &FrmHandle); if(fCmdRet==0) { //Connection succeeded .......... } |  |  |

3.1.6 CloseNetPort [disable TCP connection]

| Definition | int CloseNetPort(int FrmHandle); |  |  |
| --- | --- | --- | --- |
| Description | Disable the TCP connection to the reader and release the corresponding resources. |  |  |
| Parameters | Name | Type | Remarks |
|  | FrmHandle | Int | The handle connected to the reader, the handle obtained by opening the TCP connection. |
| Back (int) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table) |  |  |
| Reference code |  |  |  |

## 3.2. 18000-6C standard command

3.2.1. Inventory_G2 [query command]

| Definition | int Inventory_G2(BYTE * ComAddr,BYTE QValue,BYTE Session,BYTE MaskMem,BYTE *MaskAdr,BYTE MaskLen,BYTE *MaskData,BYTE MaskFlag,BYTE AdrTID,BYTE LenTID,BYTE TIDFlag,BYTE Target,BYTE InAnt,BYTE Scantime,BYTE FastFlag ,BYTE *pEPCList,BYTE *Ant,int *Totallen, int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic tag that conforms to the protocol within the valid range. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | QValue | byte | [in] | 1 byte. bit7: Statistics data packet flag. 0 – After the query is over, do not send the query process statistics data packet will not be sent; 1 – After the query is over, send the query process statistics data packet will not be sent; Bit5: Fast ID query flag, only supports some Impinj Monza series tags, used to quickly obtain the EPC and TID of the tag. 0 - Disable; 1 – Enable. Bit4～bit0: The initial Q value used when querying the EPC tag, Note: 1. The setting of the Q value should be about the number of tags in the field equal to the Q power of 2. The Q value ranges 0～15. If other values appear in the command, the parameter error information will be returned. 2. The return query speed with statistical package is slower than that without. |
|  | Session | byte | [in] | 1 byte, the session value used when querying EPC tag. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. 0xff: The reader automatically configures the session (only valid for EPC query) It is recommended to select S0 for single or a small number of tag queries. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag bit.0 – not mask; 1 – mask. |
|  | AdrTID | byte | [in] | 1 byte, query the starting word address of TID area. |
|  | LenTID | byte | [in] | 1 byte, query the number of data words in TID area, range 0~15. |
|  | TidFlag | byte | [in] | 1 byte, TID query flag bit. 0 – Read EPC; 1 – Read TID. |
|  | Target | byte | [in] | 1 byte, query the Target value of tag. 0 – Target A; 1 – Target B. |
|  | InAnt | byte | [in] | 1 byte, the antenna number to be queried this time. 0x80 – Antenna 1; 0x81 – Antenna 2; 0x82 – Antenna 3; 0x83 – Antenna 4; The single-port reader is fixed at 0x80. |
|  | scantime | byte | [in] | 1 byte, the maximum time queried by this command, range 0-255 (*100ms). When the time is 0, it means there is no limit. |
|  | FastFlag | byte | [in] | 1 byte, flag bit of fast query. 0 - Disabled, no need to specify Target, Inant, Scantime has 3 parameters, the default is 0; 1 - Enabled, it needs to specify Target, Inant, Scantime 3 parameters. |
|  | pEPCList | BYTE* | [out] | Returned tag data, format: EPC/TID length + EPC/TID number + RSSI. When there are multiple tags, several data in the same format are returned in order. |
|  | Ant | BYTE* | [out] | 1 byte, query the antenna of the tag. 4-port reader, each bit represents the antenna number, such as 0x04, the binary is 0000 0100, which means that the antenna 3 queries the tag. 12-port reader, according to the numbers 0-11 correspond to antenna 1 to antenna 12 respectively. |
|  | Totallen | Int* | [out] | Total length of data received by pEPCList. |
|  | CardNum | Int* | [out] | Number of tags found in this query. Note: When using the callback method to return, Totallen and CardNum are 0. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0x01: The command execution ends, and the queried electronic tag data is returned at the same time; 0x02: When the query time ends, the command execution is forced to exit, and the queried tag data is returned at the same time. 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected; For other values, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.2. InventoryMix_G2 [mixed query command]

| Definition | int InventoryMix_G2(BYTE * ComAddr,BYTE QValue,BYTE Session,BYTE MaskMem,BYTE *MaskAdr,BYTE MaskLen,BYTE *MaskData,BYTE MaskFlag,BYTE ReadMem,BYTE* ReadAdr,BYTE ReadLen,BYTE* Psd,BYTE Target,BYTE InAnt, BYTE Scantime,BYTE FastFlag,BYTE *pEPCList,BYTE *Ant,int *Totallen, int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic tag that conforms to the protocol within the valid range, and read the specified tag data while querying the EPC number of tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | QValue | byte | [in] | 1 byte. bit7: Statistics data packet flag. 0 – After the query is over, do not send the query process statistics data packet will not be sent; 1 – After the query is over, send the query process statistics data packet will not be sent; bit6~bit0: The initial Q value used when querying EPC tag, Note: 1. The setting of the Q value should be the number of tags in the field approximately equal to the Q power of 2. The Q value ranges 0～15. If other values appear in the command, the parameter error information will be returned. 2. The sending speed of data with statistical packet is slower than that without. |
|  | Session | byte | [in] | 1 byte, the session value used when querying the tag. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. It is recommended to select S0 for single or a small number of tag queries. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag bit.0 – not mask; 1 – mask. |
|  | ReadMem | byte | [in] | 1 byte, select the memory area to read, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | ReadAdr | BYTE* | [in] | 2 bytes, the starting word address of the memory data to be read, the high byte first. |
|  | ReadLen | byte | [in] | 1 byte, the word length of the memory data to be read. |
|  | Psd | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | Target | byte | [in] | 1 byte, query the Target value of tag. 0 – Target A; 1 – Target B. |
|  | InAnt | byte | [in] | 1 byte, the antenna number to be queried this time. 0x80 – Antenna 1; 0x81 – Antenna 2; 0x82 – Antenna 3; 0x83 – Antenna 4; The single-port reader is fixed at 0x80. |
|  | scantime | byte | [in] | 1 byte, the maximum time queried by this command, range 0-255 (*100ms). When the time is 0, it means there is no limit. |
|  | FastFlag | byte | [in] | 1 byte, flag bit of fast query. 0 - Disabled, no need to specify Target, Inant, Scantime has 3 parameters, the default is 0; 1 - Enabled, it needs to specify Target, Inant, Scantime 3 parameters. |
|  | pEPCList | BYTE* | [out] | Returned tag data, format: Rolling code + EPC/data length + EPC/data + RSSI. When there are multiple tags, several data in the same format are returned in order. The highest digit of the rolling code is 0, which means the current EPC, and the highest digit is 1, which means the memory data. The same tag data is uploaded with the EPC number first, and then the memory data. If the lower 7 bits are adjacent, it can be confirmed that it is the same data, if they are not adjacent, it is not the same tag data, the lower 7 digits of the rolling code are from 0 to 127, and the serial number is incremented by 1 each time it is uploaded. If it exceeds 127, it will start over from 0 |
|  | Ant | BYTE* | [out] | 1 byte, query the antenna of the tag. 4-port reader, each bit represents the antenna number, such as 0x04, the binary is 0000 0100, which means that the antenna 3 queries the tag. 12-port reader, according to the numbers 0-11 correspond to antenna 1 to antenna 12 respectively. |
|  | Totallen | Int* | [out] | Total length of data received by pEPCList. |
|  | CardNum | Int* | [out] | Number of tags found in this query. Note: When using the callback method to return, Totallen and CardNum are 0. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0x01: The command execution ends, and the queried electronic tag data is returned at the same time; 0x02: When the query time ends, the command execution is forced to exit, and the queried tag data is returned at the same time. 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected; For other values, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.3. InitRFIDCallBack [initialization callback]

| Definition | void InitRFIDCallBack(pRFIDCallBack pUIDback,BOOL isBackUID,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Initialize the callback function. After initializing this API, the tags obtained by the query command and the mixed query command can be uploaded through the callback method. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | pUIDback | pRFIDCallBack | [in] | Callback interface pointer typedef struct _RFIDTag { BYTE PacketParam;//Data packet type flag BYTE LEN;//Length LPCTSTR UID; //EPC or UID BYTE RSSI; BYTE ANT; int FrmHandle; }MyStruct,*RFIDTagStruct; Typedefvoid(CALLBACK*pRFIDCallBack)( RFIDTagStruct lpRfid, INT32 nEvt); |
|  | isBackUID | BOOL | [in] | Reserved. |
|  | FrmHandle | DWORD | [in] | Handle returned by the connected reader. |
| Back | No |  |  |  |
| Reference code | Refer to Programming Examples\C#\Multiple Antenna Test\MutipleAntDemo |  |  |  |

3.2.4.ReadData_G2 [command of read data]

| Definition | int ReadData_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE Mem, BYTE WordPtr, BYTE Num, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE *Data, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command reads the data in all or part of the reserved area of the tag, EPC memory, TID memory or user memory.Start reading from the specified address, in word units. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to read, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | byte | [in] | 1 byte, starting word address to read. |
|  | Num | byte | [in] | 1 byte, the word length to be read. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | Data | BYTE* | [out] | Num words, the read tag data. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.5. WriteData_G2 [command of write data]

| Definition | int WriteData_G2(BYTE *ComAdr, BYTE * EPC, BYTE Wnum, BYTE Enum, BYTE Mem, BYTE WordPtr,BYTE*Writedata,BYTE* Password, BYTE MaskMem,BYTE*MaskAdr,BYTE MaskLen,BYTE * MaskData,int * errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command can write several words to reserved memory, EPC memory, TID memory or user memory at one time. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | Wnum | byte | [in] | 1 byte, the number of words to be written, when Enum=255, the mask method is used. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag. When Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to be written, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | byte | [in] | 1 byte, the starting word address to be written. |
|  | Writedata | BYTE* | [in] | WNum bytes, the data to be written. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.6. BlockWrite_G2 [command of block write]

| Definition | int BlockWrite_G2(BYTE *ComAdr, BYTE * EPC, BYTE Wnum, BYTE Enum, BYTE Mem, BYTE WordPtr,BYTE*Writedata,BYTE* Password, BYTE MaskMem,BYTE*MaskAdr,BYTE MaskLen,BYTE * MaskData,int * errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command can write several words to reserved memory, EPC memory, TID memory or user memory at one time. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | Wnum | byte | [in] | 1 byte, the number of words to be written. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag. When Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to be written, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | byte | [in] | 1 byte, the starting word address to be written. |
|  | Writedata | BYTE* | [in] | WNum bytes, the data to be written. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.7. ExtReadData_G2 [command of extended read data]

| Definition | int ExtReadData_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE Mem, BYTE* WordPtr, BYTE Num, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE *Data, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command reads the data in all or part of the reserved area of the tag, EPC memory, TID memory or user memory.Start reading from the specified address, in word units. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to read, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | BYTE* | [in] | 2 bytes, starting word address to be read, high byte first. |
|  | Num | byte | [in] | 1 byte, the word length to be read. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | Data | BYTE* | [out] | Num words, the read tag data. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.8. ExtWriteData_G2 [command of extended write data]

| Definition | int WriteData_G2(BYTE *ComAdr, BYTE * EPC, BYTE Wnum, BYTE Enum, BYTE Mem, BYTE* WordPtr,BYTE*Writedata,BYTE* Password, BYTE MaskMem,BYTE*MaskAdr, BYTE MaskLen, BYTE * MaskData, int * errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command can write several words to reserved memory, EPC memory, TID memory or user memory at one time. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | Wnum | byte | [in] | 1 byte, the number of words to be written. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag. When Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to be written, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | BYTE* | [in] | 2 bytes, the starting word address to be written. |
|  | Writedata | BYTE* | [in] | WNum bytes, the data to be written. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.9. WriteEPC_G2 [command of broadcasting write EPC number]

| Definition | int WriteEPC_G2(BYTE * ComAddr,BYTE *Password, BYTE *EPC, BYTE ENum, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command writes the EPC number to the electronic tag. When writing, there can only be one electronic tag within the effective range of the antenna. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | EPC | BYTE* | [in] | Enum bytes, the EPC number to be written. |
|  | ENum | byte | [in] | 1 byte, the length of the tag word to be written. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.10. BlockErase_G2 [block erase command]

| Definition | int BlockErase_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE Mem, BYTE WordPtr, BYTE Num, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command erases several words in the reserved memory, EPC memory, TID memory, or user memory of the tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Mem | byte | [in] | 1 byte, select the memory area to be erased, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | WordPtr | BYTE* | [in] | 1 byte, starting word address to be erased. |
|  | Num | byte | [in] | 1 byte, word length to be erased. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.11. Lock_G2 [command of setting read and write protection status of the memory area]

| Definition | int Lock_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE select, BYTE setprotect, BYTE *Password,BYTE MaskMem,BYTE *MaskAdr,BYTE MaskLen,BYTE *MaskData, int *Errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command can set the reserved area to be readable and writable, always readable and writable, readable and writable with password, and always readable and writable, and can set EPC memory, TID memory and user memory as writable, always writable, writable with password, never writable. EPC memory, TID memory or user memory is always readable. Moreover, TID memory is read-only and can never be written. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | select | byte | [in] | 1 byte, 0x00 - Control the Kill password read and write protection settings. 0x01 – Control the access password read and write protection settings. 0x02 - Control the EPC memory read and write protection settings. 0x03 - Control the TID memory read and write protection settings. 0x04 - Control the user memory read and write protection settings. Other values are reserved. If the reader receives other values, it will return a message of parameter error. |
|  | setprotect | byte | [in] | 1 byte, When Select is 0x00 or 0x01, the meaning of the SetProtect value is as follows: 0x00 - Set to readable and writable 0x01 - Set to always readable and writable 0x02 - Set to readable and writable with password 0x03 - Set to never readable and writable When Select is 0x02, 0x03, 0x04, the meaning of the SetProtect value is as follows: 0x00 - Set to writable 0x01 - Set to always writable 0x02 - Set to writable with password 0x03 - Set to never writable. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.12. KillTag _G2 [command of destroying tag]

| Definition | int KillTag_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE *Killpwd, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to destroy the tag.Once the tag is destroyed, the commands of the memory will never be processed again. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Killpwd | BYTE* | [in] | 4 bytes, the kill password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.2.13. SingleTagInventory_G2 [single tag query command]

| Definition | int SingleTagInventory_G2(BYTE * ComAddr, BYTE* EPC, int *EPCLength, int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic tag that conforms to the protocol within the valid range. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [out] | EPCLength + 2 bytes, Return the EPC number of the tag, in the format of antenna number + EPC number + RSSI |
|  | EPCLength | Int* | [out] | 1 byte, the length of returned EPC number. |
|  | CardNum | Int* | [out] | 1 byte, the number of tags returned. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

## 3.3. Cache operation command

3.3.1. InventoryBuffer_G2 [cache query command]

| Definition | int InventoryBuffer_G2(BYTE * ComAddr, BYTE QValue, BYTE Session, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE MaskFlag, BYTE AdrTID, BYTE LenTID, BYTE TIDFlag, BYTE Target, BYTE InAnt, BYTE Scantime, BYTE FastFlag ,int * BufferCount,int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic tag that conforms to the protocol within the valid range. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | QValue | byte | [in] | 1 byte. The initial Q value used when querying EPC tag, Note: The Q value should be set so that the number of tags in the field is approximately equal to the Q power of 2. The Q value ranges 0～15. If other values appear in the command, the parameter error information will be returned. |
|  | Session | byte | [in] | 1 byte, the session value used when querying EPC tag. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. 0xff: The reader automatically configures the session (only valid for EPC query) It is recommended to select S0 for single or a small number of tag queries. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag bit.0 – not mask; 1 – mask. |
|  | AdrTID | byte | [in] | 1 byte, query the starting word address of TID area. |
|  | LenTID | byte | [in] | 1 byte, query the number of data words in TID area, range 0~15. |
|  | TidFlag | byte | [in] | 1 byte, TID query flag bit. 0 – Read EPC; 1 – Read TID. |
|  | Target | byte | [in] | 1 byte, query the Target value of tag. 0 – Target A; 1 – Target B. |
|  | InAnt | byte | [in] | 1 byte, the antenna number to be queried this time. 0x80 – Antenna 1; 0x81 – Antenna 2; 0x82 – Antenna 3; 0x83 – Antenna 4; The single-port reader is fixed at 0x80. |
|  | scantime | byte | [in] | 1 byte, the maximum time queried by this command, range 0-255 (*100ms). When the time is 0, it means there is no limit. |
|  | FastFlag | byte | [in] | 1 byte, flag bit of fast query. 0 - Disabled, no need to specify Target, Inant, Scantime has 3 parameters, the default is 0; 1 - Enabled, it needs to specify Target, Inant, Scantime 3 parameters. |
|  | BufferCount | Int* | [out] | 2 bytes, the total number of tags recorded in the cache, tags with the same EPC/TID data will be regarded as the same tag. If the cache is not cleared, the number of tags is the accumulation of the number of multiple query operations. |
|  | CardNum | Int* | [out] | 2 bytes, the times of reaching the tag in this query, regardless of whether the same tag is read for several times. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0x01: The command execution ends, and the queried electronic tag data is returned at the same time; 0x02: When the query time ends, the command execution is forced to exit, and the queried tag data is returned at the same time. 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected; For other values, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.3.2. ReadBuffer_G2 [command of getting cache data]

| Definition | int ReadBuffer_G2(BYTE *ComAdr, int *Totallen, int *CardNum, BYTE *pEPCList, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to obtain all tag information in the memory area of the reader. After the response is completed, the data in the cache is not lost and can be retrieved for several times. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Totallen | Int* | [out] | The length of pEPCList read. |
|  | CardNum | Int* | [out] | The number of tags read. |
|  | pEPCList | BYTE* | [out] | Totallen bytes, the read tag data consists of multiple tags, and the format of each tag is: ANT + LEN + EPC/TID + RSSI + Count; |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.3.3. ClearBuffer_G2 [command of clearing cache data]

| Definition | int ClearBuffer_G2(BYTE *ComAdr, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to clear cache data in cache queries. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.3.4. GetBufferCnt_G2 [command of getting the number of cached tags]

| Definition | int GetBufferCnt_G2(BYTE *ComAdr, int *Count, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to get the number of tags in the cache. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Count | Int* | [out] | The number of tags in the cache. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.3.5. SetSaveLen [command of setting the EPC/TID length of cache ]

| Definition | int SetSaveLen(BYTE *ComAdr, BYTE SaveLen, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the maximum length of EPC/TID stored in query with cache. If there is tag data in the current cache, the data in the cache will be cleared after setting the length. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | SaveLen | byte | [in] | 1 byte, specify the maximum length of the cache EPC/TID. 0x00 – 128bit length, that is, 16 bytes. 0x01 – 496bit length, that is, 62 bytes. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.3.5. GetSaveLen [command of getting the length of cached EPC/TID]

| Definition | int GetSaveLen(BYTE *ComAdr, BYTE *SaveLen, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the maximum length of EPC/TID stored in query with cache. If there is tag data in the current cache, the data in the cache will be cleared after setting the length. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | SaveLen | BYTE* | [out] | 1 byte, specify the maximum length of the cache EPC/TID. 0x00 – 128bit length, that is, 16 bytes. 0x01 – 496bit length, that is, 62 bytes. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

## 3.4. NXP tag command

3.4.1. SetPrivacyByEPC _G2 [command of setting single read protection]

| Definition | int SetPrivacyByEPC_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set read protection for the tag according to the EPC number of the electronic tag, so that the electronic tag cannot be read and written by any command, and the EPC number of the electronic tag cannot be obtained by querying the tag. Only valid for NXP's UCODE EPC G2X tags. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.4.2. SetPrivacyWithoutEPC_G2 [command of setting read protection of multiple tags]

| Definition | int SetPrivacyWithoutEPC_G2(BYTE * ComAddr, BYTE *Password, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set read protection for the electronic tags within the valid range. The difference between this command and the previous task is that when there are multiple tags in the valid range, it is impossible to know which electronic tag this command operates on. If the user wants to operate on multiple tags, the access password of the tags should preferably be the same. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.4.3. ResetPrivacy_G2 [command of unlocking read protection]

| Definition | int ResetPrivacy_G2(BYTE * ComAddr, BYTE *Password, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to unlock the tag with read protection. When this command is used, only one electronic tag to be unlocked can be placed within the effective range of the antenna. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.4.4. CheckPrivacy_G2 [command of detecting read protection]

| Definition | int CheckPrivacy_G2(BYTE * ComAddr, BYTE * readpro, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is unable to detect whether the tag supports read protection locking, but can only detect whether the tag is locked with read protection. For electronic tags that do not support read protection locking, it is agreed that they are not locked. This command can only operate on a single electronic tag to ensure that only one electronic tag exists within the effective range of the antenna. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | readpro | BYTE* | [in] | 1 byte, 0x00 - The electronic tag is not set with read protection. 0x01 - The electronic tag is set with read protection. Description: For tag that does not support read protection settings, it is considered that it is not set with read protection. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.4.5. EASConfigure_G2 [command of configuring EAS alarm]

| Definition | int EASConfigure_G2(BYTE * ComAddr,BYTE *EPC,BYTE ENum,BYTE *Password,BYTE EAS, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set or reset the EAS status bit of the electronic tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | EAS | byte | [in] | 1 byte, 0x00 – Indicating that the EAS alarm is set to be Off; 0x01 - Indicating that the EAS alarm is set to On. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.4.6. EASAlarm_G2 [command of detecting alarm]

| Definition | int EASAlarm_G2(BYTE * ComAddr, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to detect the EAS alarm of the electronic tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

## 3.5. Impinj Monza4 QT tag command

3.5.1. GetMonza4QTWorkParamter_G2 [command of read Monza4QT working parameters]

| Definition | int GetMonza4QTWorkParamter_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE *QTcontrol, int *Errorcode, int FrmHandle) ; |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read the working parameters currently set by the tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | QTcontrol | BYTE* | [out] | 1 byte, working parameter of tag. Bit0: The mirror page currently used by the tag. When Bit0 = 0, it means private; When Bit0 = 1, it means public. Bit1: Whether the tag enables distance protection. When Bit1 = 0, it means not enabled; When Bit1 = 1, it means enabled. Other values are reserved. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.5.2. SetMonza4QTWorkParamter_G2 [command of setting Monza4QT working parameters]

| Definition | int SetMonza4QTWorkParamter_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE QTcontrol, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the current working parameters of the tag. Only valid for Impinj's Monza 4QT tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | EPC | BYTE* | [in] | ENum words, the EPC number of tag. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number of the tag, when Enum=255, the mask method is used. |
|  | QTcontrol | byte | [in] | 1 byte, Working parameters of tag. Bit0: The mirror page currently used by the tag. When Bit0 = 0, it means private; When Bit0 = 1, it means public. Bit1: Whether the tag enables distance protection. When Bit1 = 0, it means not enabled; When Bit1 = 1, it means enabled. Other values are reserved. |
|  | Password | BYTE* | [in] | 4 bytes, access password of the tag. |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | Int* | [out] | When the return value is 0xFC, the tag error information is returned. Please see the ErrorCode Error Code Table. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.5.3. Inventory_QT_G2 [QT query command]

| Definition | int Inventory_QT_G2(BYTE * ComAddr, BYTE QValue, BYTE Session, BYTE Target, BYTE InAnt, BYTE Scantime, BYTE FastFlag, BYTE *pEPCList, BYTE *Ant, int *Totallen, int *CardNum, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic tag that conforms to the protocol within the valid range. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | QValue | byte | [in] | 1 byte, the initial Q value used when querying the EPC tag. Note: the setting of the Q value should be about the number of tags in the field equal to the Q power of 2. The Q value ranges 0～15. If other values appear in the command, the parameter error information will be returned. |
|  | Session | byte | [in] | 1 byte, the session value used when querying EPC tag. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. It is recommended to select S0 for single or a small number of tag queries. |
|  | Target | byte | [in] | 1 byte, query the Target value of tag. 0 – Target A; 1 – Target B. |
|  | InAnt | byte | [in] | 1 byte, the antenna number to be queried this time. 0x80 – Antenna 1; 0x81 – Antenna 2; 0x82 – Antenna 3; 0x83 – Antenna 4; The single-port reader is fixed at 0x80. |
|  | scantime | byte | [in] | 1 byte, the maximum time queried by this command, range 0-255 (*100ms). When the time is 0, it means there is no limit. |
|  | FastFlag | byte | [in] | 1 byte, flag bit of fast query. 0 - Disabled, no need to specify Target, Inant, Scantime has 3 parameters, the default is 0; 1 - Enabled, it needs to specify Target, Inant, Scantime 3 parameters. |
|  | pEPCList | BYTE* | [out] | Returned tag data, format: EPC/TID length + EPC/TID number + RSSI. When there are multiple tags, several data in the same format are returned in order. |
|  | Ant | BYTE* | [out] | 1 byte, query the antenna of the tag. 4-port reader, each bit represents the antenna number, such as 0x04, the binary is 0000 0100, which means that the antenna 3 queries the tag. 12-port reader, according to the numbers 0-11 correspond to antenna 1 to antenna 12 respectively. |
|  | Totallen | Int* | [out] | Total length of data received by pEPCList. |
|  | CardNum | Int* | [out] | Number of tags found in this query. Note: When using the callback method to return, Totallen and CardNum are 0. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0x01: The command execution ends, and the queried electronic tag data is returned at the same time; 0x02: When the query time ends, the command execution is forced to exit, and the queried tag data is returned at the same time. 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected; For other values, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.5.4. SetTagCustomFunction_G2 [command of customizing tag]

| Definition | int SetTagCustomFunction(BYTE * ComAddr, BYTE *InlayType, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the reader to start the custom working mechanism of certain types of tags to achieve specific functions of the tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | InlayType | BYTE* | [in/out] | 1 byte, tag type, the value range is 0~254. 0x00 - Indicating no tag type is specified. 0x01 – In order to start the Peek function of the Monza4QT tag (the tag status is temporarily changed from public to private), it will affect the operations such as reading data, writing data, block writing data, writing protection word and writing EPC number of the tag. Other values are reserved. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

## 3.6.18000-6B standard command

3.6.1. InventorySingle_6B [single tag query command]

| Definition | int InventorySingle_6B(BYTE *address, BYTE *Ant, BYTE* ID_6B, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command can only query a single electronic tag. If multiple tags are within the effective range of the antenna at the same time, the electronic tag may not be queried. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Ant | BYTE* | [out] | 1 byte, 0x04, binary is 0000 0100, indicating that antenna 3 has queried the tag; 0x08, binary is 0000 1000, indicating that antenna 4 has queried the tag. |
|  | ID_6B | BYTE* | [out] | 10 bytes, the UID number of the tag, the first byte is the UID length, which is 0x08. The 2nd to 9th bytes are the UID number of the tag, with the low byte first. The 10th byte is the RSSI value. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.6.2. InventoryMultiple_6B [conditional query command]

| Definition | int InventoryMultiple_6B(BYTE * ComAddr, BYTE Condition, BYTE StartAddress, BYTE mask, BYTE *ConditionContent, BYTE *Ant, BYTE* ID_6B, int *Cardnum, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command queries electronic tags according to the given conditions. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Condition | byte | [in] | 1 byte, select tag requirements. 0x00 – Equal to condition; 0x01 – Not equal to condition; 0x02 - Greater than condition; 0x03 - Less than condition. |
|  | StartAddress | byte | [in] | 1 byte, the starting address of the tag for comparison. |
|  | mask | byte | [in] | 1 byte, specify the data to compare. Each bit of Mask corresponds to one byte of ConditionContent. The highest bit of Mask (bit7) corresponds to the leftmost byte of ConditionContent. The lowest bit of Mask (bit0) corresponds to the rightmost byte of ConditionContent. The corresponding bit of Mask is 1, which means that the byte corresponding to the bit in the ConditionContent should be compared with the corresponding byte in the tag; if it is 0, it is not compared. |
|  | ConditionContent | BYTE* | [in] | 8 bytes. The condition used for comparison, that is, the data used for comparison. |
|  | Ant | BYTE* | [out] | 1 byte, the antenna of the tag is queried, such as 0x04, the binary is 0000 0100, indicating that the antenna 3 has queried the tag. |
|  | ID_6B | BYTE* | [out] | Cardnum*10 bytes, the UID of electronic tag. Each UID is 10 bytes, the first byte is the UID length, which is 0x08, the last 8 bytes are the UID number of the tag, and the last is RSSI. |
|  | CardNum | Int* | [in] | 1 byte, the number of tags read. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0x15: The command execution ends, and the queried data of electronic tag is returned at the same time; 0x16: When the query time ends, the command execution is forced to exit, and the queried tag data is returned at the same time. 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected. For other values, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.6.3. ReadData_6B [command of read data]

| Definition | int ReadData_6B(BYTE * ComAddr, BYTE* ID_6B, BYTE StartAddress, BYTE Num, BYTE *Data, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read several bytes from a specified address of the electronic tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | ID_6B | BYTE* | [in] | 8 bytes, UID number of the tag |
|  | StartAddress | byte | [in] | 1 byte. The starting address of reading data. The address range is 0 ~ 223. If the address exceeds 223, parameter error information will be returned. |
|  | Num | byte | [in] | 1 byte, the number of bytes of the data to be read. The range of Num is 1 ~ 32. If StartAddress + Num is greater than 224, or Num exceeds 32, or Num is 0, the reader will return the parameter error information. |
|  | Data | BYTE* | [out] | Num bytes, the tag data read. |
|  | ErrorCode | Int* | [out] | 1 byte, reserved. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.6.4. WriteData_6B [command of write data]

| Definition | int WriteData_6B(BYTE * ComAddr, BYTE *ID_6B, BYTE StartAddress, BYTE *Writedata, BYTE Writedatalen, BYTE *writtenbyte, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read several bytes from a specified address of the electronic tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | ID_6B | BYTE* | [in] | 8 bytes, UID number of the tag |
|  | StartAddress | byte | [in] | 1 byte. The starting address of the data to be written. The address range is 0 ~ 223. If the address exceeds 223, parameter error information will be returned. |
|  | Writedata | BYTE* | [in] | Writedatalen bytes, data of the tag to be written |
|  | Writedatalen | byte | [in] | 1 byte, the number of bytes of the data to be written. |
|  | writtenbyte | BYTE* | [out] | 1 byte, reserved. |
|  | ErrorCode | Int* | [out] | 1 byte, reserved. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.6.5. CheckLock_6B [command of detecting lock]

| Definition | int CheckLock_6B(BYTE * ComAddr, BYTE *ID_6B, BYTE StartAddress, BYTE *ReLockState, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to check whether the specified byte is locked. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | ID_6B | BYTE* | [in] | 8 bytes, UID number of the tag |
|  | StartAddress | byte | [in] | 1 byte. The address of the byte to be detected if it is locked. The address range is 0 ~ 223. If the address is out of range, parameter error information will be returned. |
|  | ReLockState | BYTE* | [out] | 1 byte, 0x00 - Not locked; 0x01 - Already locked and cannot be locked again. |
|  | ErrorCode | Int* | [out] | 1 byte, reserved. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

3.6.6. Lock_6B [command of locking]

| Definition | int Lock_6B(BYTE * ComAddr, BYTE *ID_6B, BYTE StartAddress, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to check whether the specified byte is locked. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | ID_6B | BYTE* | [in] | 8 bytes, UID number of the tag |
|  | StartAddress | byte | [in] | 1 byte. The address of the byte to be locked. The address range is 0 ~ 223. If the address is out of range, parameter error information will be returned. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No. |  |  |  |

## 3.7. Command of customizing reader

3.7.1. GetReaderInformation [command of getting reader information]

| Definition | int GetReaderInformation(BYTE* ComAdr,BYTE* VersionInfo, BYTE* ReaderType, BYTE* TrType, BYTE* dmaxfre, BYTE* dminfre, BYTE* powerdBm, BYTE* ScanTime, BYTE*Ant, BYTE*BeepEn, BYTE*OutputRep, BYTE*CheckAnt, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader information, including reader address (ComAdr) and software version (VersionInfo) etc. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | 1 byte, use the reader address returned by the connected reader. |
|  | VersionInfo | BYTE* | [out] | 2 bytes, reader version information. The first byte is the version number, and the second byte is the handle of the subversion number. |
|  | ReaderType | BYTE* | [out] | 1 byte, type code of the reader. |
|  | TrType | BYTE* | [out] | 1 byte, the protocols supported by the reader. |
|  | dmaxfre | BYTE* | [out] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current maximum frequency of the reader. |
|  | dminfre | BYTE* | [out] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current minimum frequency of the reader. |
|  | PowerDbm | BYTE* | [out] | 1 byte, the output power of the reader. The range is 0-30 in dBm. |
|  | scantime | BYTE* | [out] | 1 byte, query time. After the reader receives the query command, it will reply to the master computer within the query time. |
|  | Ant | BYTE* | [out] | 1 byte, antenna configuration information. |
|  | BeepEn | BYTE* | [out] | 1 byte, buzzer switch information. |
|  | OutputRep | BYTE* | [out] | 1 byte, notice output port parameters. |
|  | CheckAnt | BYTE* | [out] | 1 byte, antenna detection information. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

Note: The frequency band settings are as follows:

| MaxFre(Bit7) | MaxFre(Bit6) | MinFre(Bit7) | MinFre(Bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

3.7.2 SetAddress [command of setting reader address]

| Definition | int SetAddress(BYTE * ComAddr, BYTE ComAdrData, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | After executing this command, the reader will change the address of the reader to the value given by the user, and write and save this value into the EEPROM. The factory default value is 0x00. The modification range allowed by the user is 0x00~0xFE. When the value written by the user is 0xFF, the reader will automatically return to the default value of 0x00. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | ComAdrData | byte | [in] | 1 byte, the new address after modification. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.3 SetInventoryScanTime [command of setting the maximum response time of query command]

| Definition | int SetInventoryScanTime(BYTE *address, BYTE ScanTime, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the maximum response time of the query command. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | scantime | byte | [in] | 1 byte, query command response time, range 0-255. When set to 0, this parameter is invalid and waits until the round is read. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.4. SetRfPower [command of setting power]

| Definition | int SetRfPower(BYTE *address, BYTE PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the power of reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | PowerDbm | byte | [in] | 1 byte, reader power, range 0~30dBm. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.5. SetRegion [command of setting working frequency]

| Definition | int SetRegion(BYTE *address, BYTE dmaxfre, BYTE dminfre, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command sets the upper limit frequency and lower limit frequency of the reader. The upper limit frequency must be greater than or equal to the lower limit frequency. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | dmaxfre | byte | [in] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current maximum frequency of the reader. |
|  | dminfre | byte | [in] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current minimum frequency of the reader. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

Note: The frequency band settings are as follows:

| MaxFre(Bit7) | MaxFre(Bit6) | MinFre(Bit7) | MinFre(Bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

The calculation formula of each frequency band:

Chinese band2: Fs = 920.125 + N * 0.25 (MHz), where N∈[0, 19].

US band: Fs = 902.75 + N * 0.5 (MHz), where N∈[0,49].

Korean band: Fs = 917.1 + N * 0.2 (MHz), where N∈[0, 31].

EU band:Fs = 865.1 + N*0.2(MHz), where N∈[0, 14].

Ukraine band:Fs = 868.0 + N*0.1(MHz), where N∈[0, 6].

Chinese band1: Fs = 840.125 + N * 0.25 (MHz), where N∈[0, 19].

US band3: Fs = 902 + N * 0.5 (MHz), where N∈[0,52].

3.7.6. SetBaudRate [command of setting baud rate]

| Definition | int SetBaudRate(BYTE * ComAddr, BYTE baud, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the communication baud rate of the reader |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Baud | byte | [in] | 1 byte, baud rate. 0 – 9600bps; 1 – 19200bps; 2 – 38400bps; 5 – 57600bps; 6 – 115200bps. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.7. BuzzerAndLEDControl [acousto-optic control command]

| Definition | int BuzzerAndLEDControl(BYTE * ComAddr, BYTE AvtiveTime, BYTE SilentTime, BYTE Times, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to control the LED lights and buzzer to flash and beep according to certain rules. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | AvtiveTime | byte | [in] | 1 byte, LED light on and buzzer beeping time (ActiveT*50ms), the default value is zero. 0< =ActiveT< =255. |
|  | SilentTime | byte | [in] | 1 byte, LED light and buzzer silent time (SilentT *50ms), the default value is zero. 0< =SilentT< =255. |
|  | Times | byte | [in] | 1 byte, LED light on and buzzer beeping times (0< =Times< =255) The default value is zero. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.8. SetAntennaMultiplexing [command of configuring antenna]

| Definition | int SetAntennaMultiplexing(BYTE *ComAdr, BYTE Ant, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used for 4-port reader antenna configuration. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Ant | byte | [in] | 1 byte, antenna configuration information. At least one antenna must be configured. Bit0: Antenna 1 configuration bit. Bit0=0 Antenna 1 is off; Bit0=1 Antenna 1 is on. Bit1: Antenna 2 configuration bit. Bit1=0 Antenna 2 is off; Bit1=1 Antenna 2 is on. Bit2: Antenna 3 configuration bit. Bit2=0 Antenna 3 is off; Bit2=1 Antenna 3 is on. Bit3: Antenna 4 configuration bit. Bit3=0 Antenna 4 is off; Bit3=1 Antenna 4 is on. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.9. SetBeepNotification [command of setting buzzer]

| Definition | int SetBeepNotification(BYTE *ComAdr,BYTE BeepEn, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the buzzer switch. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | BeepEn | byte | [in] | 1 byte, 0x00 - buzzer off; 0x01 - Buzzer on. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.10. SetRelay [command of controlling relay]

| Definition | int SetRelay(BYTE *ComAdr, BYTE RelayTime, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to control the relay to close. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | RelayTime | byte | [in] | 1 byte, relay closing time (1~255)*50ms. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.11. GetGPIOStatus [command of read GPIO status]

| Definition | int GetGPIOStatus(BYTE *ComAdr, BYTE *OutputPin, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read the output of general GPIO port, the default output is high level. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | OutputPin | BYTE* | [out] | 1 byte, Bit0 represents the pin status of IN1, Bit4-Bit5 represents the status of Out1-Out2 respectively, and other bits are reserved. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.12. SetGPIO [command of controlling GPIO]

| Definition | int SetGPIO(BYTE *ComAdr, BYTE OutputPin, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the output state of the GPIO port. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | OutputPin | byte | [in] | 1 byte, GPIO port (Out1-Out2 pin) output status. Bit0-Bit1 control the Out1-Out2 pins respectively, and Bit2-Bit7 are reserved. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.13. GetSeriaNo [command of getting reader serial number]

| Definition | int GetSeriaNo(BYTE * ComAddr, BYTE* SeriaNo, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to get the 4-byte unique serial number of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | SeriaNo | BYTE* | [out] | 4 bytes, serial number of the reader. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.14. SetCheckAnt [command of detecting antenna]

| Definition | int SetCheckAnt(BYTE *address, BYTE CheckAnt, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set whether to perform antenna detection during the process of reading and writing tags. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | CheckAnt | byte | [in] | 1 byte, antenna detection switch. 0x00 - Disable antenna detection during tag reading and writing. 0x01 – Enable antenna detection during tag reading and writing. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.15. WriteRfPower [command of setting write power]

| Definition | int WriteRfPower(BYTE * ComAddr, BYTE PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the power when writing tags. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | PowerDbm | byte | [in] | 1 byte, the write power of the reader, 0-30; bit7=0 – Disabled; bit7=1 – Enable. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.16. ReadRfPower [command of read the write power]

| Definition | int WriteRfPower(BYTE * ComAddr, BYTE PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read the power when writing the tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | PowerDbm | BYTE* | [out] | 1 byte, the write power of the reader, 0-30; bit7=0 – Disabled; bit7=1 – Enable. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.17. RetryTimes [command of setting write retry times]

| Definition | int RetryTimes(BYTE * ComAddr, BYTE *Times, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read the power when writing the tag. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Times | BYTE* | [out] | 1 byte, bit7=1 - Set the retry times; bit7=0 – Read the retry times; range 0-7. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.18. SetReadMode [command of setting real-time mode]

| Definition | int SetReadMode(BYTE *ComAdr,BYTE ReadMode ,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the real-time query mode of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | ReadMode | byte | [in] | 1 byte, 0 – Answer mode; 1 – Real-time query mode; 2 – Trigger mode. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.19. SetReadParameter [command of setting working parameters in real-time mode]

| Definition | int SetReadParameter(BYTE *ComAdr, BYTE *Parameter, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE MaskFlag, BYTE AdrTID, BYTE LenTID, BYTE TIDFlag, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the working parameters of the reader in real-time query mode. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Parameter | BYTE* | [in] | 5 bytes, Parameter[0]: Tag type for automatic real-time query. If other values are present in the command, the parameter error information is returned. 0 – EPC C1G2 (ISO18000-6C) tag. 1 – ISO18000-6B tag. Parameter[1]:The reader automatically queries the interval time of tags in real time. 0x00 – 10ms; 0x01 – 20ms; 0x02 – 30ms; 0x03 – 50ms; 0x04 - 100ms. Parameter[2]: Tag filter time.Take 1s as the unit, the parameter range is 0～255s.This parameter means that in the automatic real-time query mode, if the reader reads the same tag multiple times within the filtering time, it will only upload it once. If this parameter is 0, it means that tag filtering is not enabled. Parameter[3]: 1 byte, the initial Q value used when querying the EPC tag. The Q value should be set so that the number of tags in the field is approximately equal to 2Q. The Q value ranges from 0 to 15. If other values appear in the command, the parameter error information will be returned. Parameter[4]: Session value used when querying EPC tags. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. 0xff: The reader automatically configures the Session (only valid for EPC query). |
|  | MaskMem | byte | [in] | 1 byte, mask area. 1 – EPC memory area; 2 – TID memory area; 3 – User memory area. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, the starting bit address of the mask (unit: Bits). Range 0~16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of the mask (unit: Bits). |
|  | MaskData | BYTE* | [in] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | AdrTID | byte | [in] | 1 byte, query the starting word address of TID area. |
|  | LenTID | byte | [in] | 1 byte, query the number of data words in TID area, range 0~15. |
|  | TidFlag | byte | [in] | 1 byte, TID query flag bit. 0 – Read EPC; 1 – Read TID. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.20. GetReadParameter [command of read working parameters in real-time mode]

| Definition | int GetReadParameter (BYTE *ComAdr,BYTE* Parameter,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the real-time query mode of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | Parameter | BYTE* | [out] | Parameter in order is: ReadMode: 1 byte, the current working mode of the reader. TagProtocol: 1 byte, the tag type of the automatic real-time query mode. ReadPauseTime: 1 byte, the interval time for the reader to automatically query the tag in real time. FliterTime: 1 byte, the tag filtering time in automatic real-time query mode. QValue: 1 byte, the initial Q value used when automatically querying the EPC tag in real time. Session: 1 byte, the Session value used when automatically querying the EPC tag in real time. MaskMem, MaskAdr, MaskLen, MaskData: In automatic real-time query mode, the reader queries the EPC C1G2 tag mask condition parameters.Among them, MaskMem is one byte; MaskAdr is two bytes, with the high byte first; MaskLen is one byte; MaskData is fixed to 32 bytes, and the content beyond that indicated by MaskLen is zero. AdrTID: Automatically query the starting word address of the TID area in real time. LenTID: Automatically query the number of data words in the TID area in real time. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.21. SetDRM [set DRM]

| Definition | int SetDRM(BYTE * ComAddr, BYTE DRM, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the DRM state. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | DRM | byte | [in] | 1 byte, DRM status, 0 - Off; 1 – On. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.22. GetDRM [read DRM]

| Definition | int GetDRM(BYTE * ComAddr, BYTE* DRM, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read DRM status. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | DRM | BYTE* | [out] | 1 byte, DRM status, 0 - Off; 1 – On. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.23. GetReaderTemperature [read temperature]

| Definition | int GetReaderTemperature (BYTE * ComAddr, BYTE* PlusMinus, BYTE* Temperature, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read the current temperature of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | PlusMinus | BYTE* | [out] | 1 byte, temperature status, 0 – Minus zero; 1 – Above zero. |
|  | Temperature | BYTE* | [out] | 1 byte, temperature value, unit °c |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.24. MeasureReturnLoss [measure return loss]

| Definition | int MeasureReturnLoss(BYTE * ComAddr, BYTE* TestFreq, BYTE Ant, BYTE* ReturnLoss,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to measure the return loss of the specified port of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | TestFreq | BYTE* | [in] | 4 bytes, measurement frequency. |
|  | Ant | byte | [in] | 1 byte, measurement antenna port, 0 – Antenna 1; 1 – Antenna 2. 2 – Antenna 3; 3 – Antenna 4. |
|  | ReturnLoss | BYTE* | [out] | 1 byte, return loss, in dB. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.25. SetNotificationPulseOutput [command of notice output port]

| Definition | int SetNotificationPulseOutput(BYTE *ComAdr, BYTE OutputRep, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set whether to output a 2ms low pulse at the corresponding port when the tag is queried. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | ComAddr | BYTE* | [in/out] | Use the reader address returned by the connected reader. |
|  | OutputRep | byte | [in] | 1 byte, Bit0: Correspond to antenna 1 and Out1 pin. When Bit0=1, when Antenna 1 finds the tag, Out1 pin outputs a low pulse of 2ms; when Bit0=0, no signal is output. Bit1: Correspond to antenna 2 and Out2 pin. When Bit1=1, when Antenna 2 finds the tag, Out2 pin outputs a low pulse of 2ms; when Bit1=0, no signal is output. Bit2: Correspond to antenna 3 and Out3 pin. When Bit2=1, when Antenna 3 finds the tag, Out3 pin outputs a low pulse of 2ms; when Bit2=0, no signal is output. Bit3: Correspond to antenna 4 and Out4 pin. When Bit3=1, when Antenna 4 finds the tag, Out4 pin outputs a low pulse of 2ms; when Bit3=0, no signal is output. Other bits are reserved, and the default value is 0. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

3.7.26. ReadActiveModeData [get active uploaded data]

| Definition | int ReadActiveModeData(BYTE * LoadData, int *Datalength, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to obtain the data uploaded by the reader in active mode or real-time mode. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | LoadData | BYTE* | [out] | Datalength bytes, the raw data uploaded, please refer to the User Manual for the format. |
|  | Datalength | BYTE* | [out] | The length in bytes of LoadData. |
|  | FrmHandle | Int | [in] | Handle returned by the connected reader. |
| Back (int) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Return Value Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

# Appendix 1.Return Value Error Code Table

| Error Code | Description |
| --- | --- |
| 0x00 | No error, indicating API call successful. |
| 0x01 | Return before the query time expires. |
| 0x02 | The specified query time overflow. |
| 0x05 | Access password error. |
| 0x09 | Destroy password error. |
| 0x0A | Destroy password shall not be all 0s. |
| 0x0B | Electronic tag does not support this command. |
| 0x0C | For this command, the access password shall not be 0. |
| 0x0D | The electronic tag has been set with reading protection and cannot be set again. |
| 0x0E | The electronic tag is not set with reading protection and does not need to be unlocked. |
| 0x10 | Byte space locked and writing failed. |
| 0x11 | Can not be locked. |
| 0x12 | Already locked and cannot be locked again. |
| 0x13 | The parameter saving failed, but the set value is valid until the read-write module is powered off. |
| 0x14 | Unable to adjust. |
| 0x15 | Return before the query time expires. |
| 0x17 | There is message after this message. |
| 0x18 | The storage space of the read-write module is full. |
| 0x19 | Electron does not support this command or the access password shall not be 0 |
| 0xF8 | Antenna detection error. |
| 0xF9 | Command execution error. |
| 0xFA | There is an electronic tag, but the communication is poor and cannot be operated. |
| 0xFB | No electronic tag is available for operating. |
| 0xFC | Electronic tag returns an error code. |
| 0xFD | Command length error. |
| 0xFE | Illegal order. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |
| 0x33 | Communication is busy and the device is executing other commands. |
| 0x35 | Port is open |
| 0x37 | Invalid handle. |

# Appendix 2.ErrorCode Error Code Table

| Error Code | Description |
| --- | --- |
| 0x00 | Other errors, all catch errors not covered by other codes. |
| 0x03 | Memory overrun or unsupported PC value, the specified storage location does not exist or PC value not supported by the tag. |
| 0x04 | Memory lock, memory location lock is permanently locked and cannot be written. |
| 0x0B | Insufficient power supply, insufficient tag power to perform storage and writing operations. |
| 0x0F | Non-specific error, the tag does not support specific error code. |