UHF2000.ocx Control Function Library

# Table of Contents 1. Control Call Process

Table of Contents	2

1. Control call flow	3

1.1 Control registration	3

1.2 Connection and closing of the reader	3

1.3 Acquisition of tag	3

2. Programming Interface	5

2.1 Connecting and closing the interface of the reader	5

2.1.1 RFID_ComOpen	5

2.1.2 RFID_OpenTcp	6

2.1.3 RFID_CloseCom	7

2.1.4 RFID_CloseTcp	8

2.2 18000-6C protocol interface	9

2.2.1 RFID_Inventory	9

2.2.2 RFID_Read	10

2.2.3 RFID_Write	11

2.2.4 RFID_WriteEPC	12

2.3 Command of customizing reader	13

2.3.1 RFID_GetSeriaNo	13

2.3.2 RFID_SetRegion.	14

2.3.3 RFID_SetRfPower	15

2.3.4 RFID_SetInventoryScanTime	16

2.3.5 RFID_SetBaudRate	17

2.3.6 RFID_SetAntenna	18

2.3.7 RFID_CheckAnt	19

2.3.8 RFID_Beep	20

2.3.9 RFID_WriteRfPower	21

Appendix 1. Return Value Error Code Table	22

## 1.1 Control registration

Copy the library files UHFReader288MP.dll and UHF2000.ocx in the ".\SDK\Development Class Library\ocx" directory of the development kit, as shown in Figure 1-1. For 32-bit system, place it in the C:\WINDOWS\system32 directory (for 64-bit systems, place it in C:\WINDOWS\SysWOW64).

Figure 1-1

After copying, for 32-bit operating system, directly run "Control Registration (win32).bat", for 64-bit system, run "Control Registration (win64).bat" in administrator mode. After operation, if the prompt message shown in Figure 1-2 appears, it indicates that the control registration is successful. Some computers may prompt unsuccessful, but it can be used normally.

Figure 1-2

## 1.2 Connection and closing of the reader

Before doing any operations on the reader, the reader must be connected. According to the specific situation of the reader, call RFID_ComOpen (serial communication) or RFID_TcpOpen (network communication) to connect.

After completing all operations on the reader, call RFID_ComClose or RFID_TcpClose to close the connection and release the occupied memory space.

## 1.3 Acquisition of tag

After the reader is connected successfully, call RFID_Inventory to obtain the EPC or TID of the tag. The return format of each tag: byte length + EPC/TID + RSSI.

For example: OCE200123412341234444455558D0A11112222333344444555584

First take the first length byte, indicating that the following tag has 12 bytes, and counting one byte of RSSI, the format data of the first gag can be obtained as OCE200123412341234444455558D;

The format data of the second tag is 0A1111222233334444555584

Getting rid of the length and RSSI, the real EPC number data of the two tags are E20012341234123444445555 and 11112222333344445555 respectively.

# 2. Programming interface

## 2.1 Connecting and closing the interface of the reader

2.1.1 RFID_ComOpen

| Definition | long RFID_ComOpen(long Port, long Baud); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable the device by serial port |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | Port | long | [in] | Serial port number connected to the reader.1-COM1; 2-COM2… |
|  | Baud | long | [in] | Serial communication baud rate.0-9600; 1-19200; 2-38400; 5-57600; 6-115200. |
| Back (LONG) | Successful: 0; Failed: Non-0; (see the Return Value Error Code Table). |  |  |  |
| Reference code |  |  |  |  |

2.1.2 RFID_OpenTcp

| Definition | long RFID_OpenTcp(LPCSTR ipAddr , long nPort); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable the device by network. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | nPort | long | [in] | Port No. |
|  | ipAddr | LPCSTR | [in] | IP address |
| Back (LONG) | Successful: 0; Failed: Non-0 (see Error Code Table); |  |  |  |
| Reference code |  |  |  |  |

2.1.3 RFID_CloseCom

| Definition | long RFID_ComClose(); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Release the serial port connected to the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
| Back (LONG) | Successful: 0; Failed: Non-0 (see Error Code Table); |  |  |  |
| Reference code |  |  |  |  |

2.1.4 RFID_CloseTcp

| Definition | long RFID_TcpClose(); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Release the network port connected to the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
| Back (LONG) | Successful: 0; Failed: Non-0 (see Error Code Table); |  |  |  |
| Reference code |  |  |  |  |

## 2.2 18000-6C protocol interface

2.2.1 RFID_Inventory

| Definition | LPCSTR RFID_Inventory(long QValue, long Session, long TidFlag, long Ant, long Target, long scantime); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic tag that conforms to the protocol within the valid range. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | QValue | long | [in] | The initial Q value used when querying the EPC tag, Note: 1. The setting of the Q value should be about the number of tags in the field equal to the Q power of 2. The Q value ranges 0～15. If other values appear in the command, the parameter error information will be returned. 2. The return query speed with statistical package is slower than that without. |
|  | Session | long | [in] | Session value used when querying EPC tags. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. 0xff: The reader automatically configures the session (only valid for EPC query) When Session is set to 255, Ant, Target, scantime Parameter is invalid. It is recommended to select S0 for single or a small number of tag queries. |
|  | TidFlag | long | [in] | Query the TID flag, 0 - query EPC; 1 - query TID. |
|  | Ant | long | [in] | The antenna No. to be queried this time. 0x80 – Antenna 1; 0x81 – Antenna 2; 0x82 – Antenna 3; 0x83 – Antenna 4; |
|  | Target | long | [in] | Query the Target value of the tag. 0 – Target A; 1 – Target B. |
|  | scantime | long | [in] | The maximum time queried by this command, the range is 0-255 (*100ms). When the time is 0, it means there is no limit. |
| Back (LPCSTR ) | If no tag is found, return empty; The query finds the label and returns a hexadecimal string, the format is such as 1.3 Get the tag |  |  |  |
| Reference code | No |  |  |  |

2.2.2 RFID_Read

| Definition | LPCSTR RFID_Read (LPCSTR EPC,long WordAddr, long ReadNum, long Mem); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command reads the data in all or part of the reserved area of the tag, EPC memory, TID memory or user memory. Start reading from the specified address, in word units. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | EPC | LPCSTR | [in] | EPC number of tag |
|  | WordAddr | long | [in] | The starting word address to read . |
|  | ReadNum | long | [in] | The length of words to read. |
|  | Mem | long | [in] | Select the memory area to read, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
| Back (LPCSTR ) | When successful, the read data (hexadecimal string) is returned; when failed, an empty string is returned. |  |  |  |
| Reference code | No |  |  |  |

2.2.3 RFID_Write

| Definition | long RFID_Write (LPCSTR EPC, long WordAddr, long Mem, LPCSTR Wdata); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command can write several words to reserved memory, EPC memory, TID memory or user memory at one time. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | EPC | LPCSTR | [in] | EPC number of tag |
|  | WordAddr | long | [in] | The starting word address to read . |
|  | Mem | long | [in] | The memory area to write to, 0x00 - reserved area; 0x01 – EPC memory area; 0x02 – TID memory area; 0x03 – User memory area. Other values are reserved. If other values are present in the command, the parameter error information is returned. |
|  | Wdata | LPCSTR | [in] | Data to be written, hexadecimal string, written in characters. |
| Back (LONG) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

2.2.4 RFID_WriteEPC

| Definition | long RFID_WriteEPC(LPCSTR EPC); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command writes the EPC number to the electronic tag. When writing, there can only be one electronic tag within the effective range of the antenna. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | EPC | LPCSTR | [in] | The EPC number to be written, a hexadecimal string, written in characters. |
| Back (LPCSTR ) | When successful, the read data (hexadecimal string) is returned; when failed, an empty string is returned. |  |  |  |
| Reference code | No |  |  |  |

## 2.3 Command of customizing readers

2.3.1 RFID_GetSeriaNo

| Definition | LPCSTR RFID_GetSeriaNo() |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Get the serial number of reader |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
| Back (LONG) | When successful, the read serial number (4-byte hexadecimal string) is returned; when failed, an empty string is returned. |  |  |  |
| Reference code | No |  |  |  |

2.3.2 RFID_SetRegion.

| Definition | long RFID_SetRegion (long dmaxfre, long dminfre) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set spectrum |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | dmaxfre | long | [in] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current maximum frequency of the reader. |
|  | dminfre | long | [in] | 1 byte, Bit7-Bit6 are used for frequency band setting; Bit5-Bit0 represents the current minimum frequency of the reader. |
| Back (LONG) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

Note: The frequency band settings are as follows:

| MaxFre(Bit7) | MaxFre(Bit6) | MinFre(Bit7) | MinFre(Bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

The calculation formula of each frequency band:

Chinese band2: Fs = 920.125 + N * 0.25 (MHz), where N∈[0, 19].

US band: Fs = 902.75 + N * 0.5 (MHz), where N∈[0,49].

Korean band: Fs = 917.1 + N * 0.2 (MHz), where N∈[0, 31].

EU band: Fs = 865.1 + N*0.2(MHz), where N∈[0, 14].

Ukraine band: Fs = 868.0 + N*0.1(MHz), where N∈[0, 6].

Chinese band1: Fs = 840.125 + N * 0.25 (MHz), where N∈[0, 19].

US band3: Fs = 902 + N * 0.5 (MHz), where N∈[0,52].

2.3.3 RFID_SetRfPower

| Definition | long RFID_SetRfPower (long PowerDbm) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set power |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | PowerDbm | long | [in] | 1 byte, reader power, range 0~30dBm. |
| Back (LONG) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

2.3.4 RFID_SetInventoryScanTime

| Definition | long RFID_SetInventoryScanTime (long ScanTime) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the maximum response time of the query command |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | scantime | long | [in] | 1 byte, query command response time, range 0 to 255, unit 100ms. When set to 0, this parameter is invalid and waits until the round is read. |
| Back (LPCSTR ) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

2.3.5 RFID_SetBaudRate

| Definition | long RFID_SetBaudRate (long baud) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the communication baud rate of the reader. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | Baud | long | [in] | 1 byte, baud rate. 0 – 9600bps; 1 – 19200bps; 2 – 38400bps; 5 – 57600bps; 6 – 115200bps. |
| Back (LONG) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

2.3.6 RFID_SetAntenna

| Definition | long RFID_SetAntenna(long Antenna) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used for 4-port reader antenna configuration. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | Antenna | long | [in] | 1 byte, antenna configuration information. At least one antenna must be configured. Bit0: Antenna 1 configuration bit. Bit0=0 Antenna 1 is off; Bit0=1 Antenna 1 is on. Bit1: Antenna 2 configuration bit. Bit1=0 Antenna 2 is off; Bit1=1 Antenna 2 is on. Bit2: Antenna 3 configuration bit. Bit2=0 Antenna 3 is off; Bit2=1 Antenna 3 is on. Bit3: Antenna 4 configuration bit. Bit3=0 Antenna 4 is off; Bit3=1 Antenna 4 is on. |
| Back (LONG) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

2.3.7 RFID_CheckAnt

| Definition | long RFID_CheckAnt(long CheckAnt) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set whether to perform antenna detection during the process of reading and writing tags. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | Antenna | long | [in] | 1 byte, antenna detection switch. 0x00 - Disable antenna detection during tag reading and writing. 0x01 – Enable antenna detection during tag reading and writing. |
| Back (LONG) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

2.3.8 RFID_Beep

| Definition | long RFID_Beep(long BeepEn) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the buzzer switch. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | BeepEn | long | [in] | 1 byte, 0x00 - buzzer off; 0x01 - Buzzer on. |
| Back (LONG) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

2.3.9 RFID_WriteRfPower

| Definition | long RFID_WriteRfPower (long PowerDbm) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to set the power when writing tags. |  |  |  |
| Parameters | Name | Type | Direction | Remarks |
|  | PowerDbm | long | [in] | 1 byte, the write power of the reader, 0-30; bit7=0 - Disable; bit7=1 – Enable. |
| Back (LONG) | 0: Indicate successful. Not equal to 0: Indicate failed, please see the Error Code Table. |  |  |  |
| Reference code | No |  |  |  |

# Appendix 1.Return Value Error Code Table

| Error Code | Description |
| --- | --- |
| 0x00 | No error, indicating API call successful. |
| 0x01 | Return before the query time expires. |
| 0x02 | The specified query time overflow. |
| 0x05 | Access password error. |
| 0x09 | Destroy password error. |
| 0x0A | Destroy password shall not be all 0s. |
| 0x0B | Electronic tag does not support this command. |
| 0x0C | For this command, the access password shall not be 0. |
| 0x0D | The electronic tag has been set with reading protection and cannot be set again. |
| 0x0E | The electronic tag is not set with reading protection and does not need to be unlocked. |
| 0x10 | Byte space locked and writing failed. |
| 0x11 | Can not be locked. |
| 0x12 | Already locked and cannot be locked again. |
| 0x13 | The parameter saving failed, but the set value is valid until the read-write module is powered off. |
| 0x14 | Unable to adjust. |
| 0x15 | Return before the query time expires. |
| 0x17 | There is message after this message. |
| 0x18 | The storage space of the read-write module is full. |
| 0x19 | Electron does not support this command or the access password shall not be 0 |
| 0xF8 | Antenna detection error. |
| 0xF9 | Command execution error. |
| 0xFA | There is an electronic tag, but the communication is poor and cannot be operated. |
| 0xFB | No electronic tag is available for operating. |
| 0xFC | Electronic tag returns an error code. |
| 0xFD | Command length error. |
| 0xFE | Illegal order. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |
| 0x33 | Communication is busy and the device is executing other commands. |
| 0x35 | Port is open |
| 0x37 | Invalid handle. |